INFORMAL AUTOMATIC TRANSLATION

Consolidated version after 07 January 2020

Order issued: Tar 2014-03-12, i.e. 2014-02982

MINISTER OF ENVIRONMENT OF THE REPUBLIC OF LITHUANIA

ORDER

On the approval of rules ON the granting, UPDATING and revocation of pollution permits

6 March 2014 No D1-259

Vilnius

Pursuant to Article 192 of the Law of the Republic of Lithuania on Environmental Protection, paragraph 3 of Article 9 and paragraph 2 of Article 18 of the Law of the Republic of Lithuania on Water:

Amendments to the preamble:

No D1-357, 2019-06-11 published TAR 2019-06-11, i.e. 2019-09454

1.I approve the Rules on the granting, updating and revocation of pollution permits (hereinafter referred to as the Rules) (attached).

2.Ifind that:

2.1. Allowances for the operation of installations meeting the criteria set out in Annex 1 to the Rules shall be issued or replaced with effect from 15 April 2014 upon application in accordance with the procedure laid down in the Rules;

2.2. Installations complying with the criteria specified in Annex 1 to the Rules whose operation before 14 April 2014 has been granted integrated pollution prevention and control permits in accordance with Annex 2 to the Rules for Issuing, Renewal and Revocation of Integrated Pollution Prevention and Control Permits approved by Order No 80 of the Minister of Environment of the Republic of Lithuania on 27 February 2002 (hereinafter: ‘IPPC rules 2002’, IPPC permit) shall be subject to the conditions of conversion of the permit as specified in Article 192 of the Law of the Republic of Lithuania on Environmental Protection as of 15 April 2014;

2.3. applications for the issue or renewal of an IPPC permit or applications for adjustment of the IPPC permit in accordance with the requirements of the IPPC rules of 2002 shall be submitted to regional environmental departments before 14 April 2014;

2.4. if an application for an IPPC permit has been submitted before 14 April 2014, or an application for an IPPC permit has been submitted in accordance with the criteria specified in Annex 2 to the 2002 IPPC Regulations, the application procedure shall be completed in accordance with the requirements of the IPPC Regulations of 2002 and shall not be longer than 30 June and the permit shall be issued in accordance with the requirements of the Rules;

2.5. point 2.4 of this Order shall not apply to installations which, according to the criteria set out in Annex 2 to the 2002 IPPC Regulations, are subject to an IPPC permit but do not meet the criteria set out in Annex 1 to the Rules. In that case, the procedures for issuing, renewing or revising the IPPC permit shall be completed in accordance with the procedure laid down in the IPPC rules for 2002 and shall not extend beyond 30 June 2014;

2.6. procedures for revocation of IPPC permits issued in accordance with Annex 2 to the 2002 IPPC Regulations shall be carried out in accordance with the procedure laid down in the Rules in accordance with the procedure laid down in the Rules of Procedure of the Republic of Lithuania Law on Environmental Protection as of 1 July 2014;

2.7. IPPC permits issued in application of the criteria of Annex 2 to the 2002 IPPC Regulations shall continue to be valid, shall be treated as pollution permits and shall be subject to the requirements of the Regulations as of 1 July 2014, except for the cases specified in subparagraph 2.5 of this Order; Compliance with their conditions shall be controlled in accordance with the requirements and conditions laid down in the Regulations for the operation of the installation;

Amendments to point.:

No D1-401, 2017-05-11 published TAR 2017-05-12, i.e. 2017-08065

2.8. applications for the issue or renewal of a permit in accordance with the requirements of the Rules shall be submitted to regional environmental protection departments until 30 June 2014. As of 1 July 2014, the procedures for granting, changing or withdrawing a permit in accordance with the requirements of the Rules shall continue to be implemented and completed by the Environmental Protection Agency;

2.9. where the regional environmental protection department fails to complete the procedures referred to in subparagraphs 2.4 and 2.8 of this Order by 1 July 2014, not later than within 3 working days, submit to the Environmental Protection Agency all documents relating to the procedures for the issuance, replacement and/or revocation of the permit;

2.10. Installations meeting the criteria set out in Annex 1 to the Rules whose operation did not require an IPPC permit issued in accordance with the IPPC Regulations 2002 before 15 April 2014 shall be granted a permit before 1 November 2014 in accordance with the procedure laid down in the Rules.

2.11.ship recycling facilities as defined in Article 3(1)(7) of Regulation (EU) No 1257/2013 of the European Parliament and of the Council of 20 November 2013 on ship recycling and amending Regulation (EC) No 1013/2006 and Directive 2009/16/EC (OJ 2013 L 330, p.1) (hereinafter referred to as the “Regulation (EU) No 1257/2013”) shall be replaced by the IPK permits set out in Articles 2(1) and 1257 of Regulation (EU) No 2013 (hereinafter referred to as the “installation”) with valid IPPC permits for ship recycling activities issued before 30 June 2014 in accordance with the requirements of Articles 2 and 1257 of IPK Regulation 2013. These IPPC permits are to be replaced by 1 March 2015. Special conditions for ship recycling facilities are to be laid down in accordance with the procedure laid down by the Rules.

The following subparagraph is added:

No D1-1049, 2014-12-19 published TAR 2014-12-22, i.e. 2014-20406

2.12. for ship recycling installations which hold valid emission permits with a specific part of the permit “Waste treatment (recovery or disposal, including preparation for recovery or disposal) and storage”, allowing recycling of ships issued or modified after 15 April 2014 as waste, the allowances shall be replaced in accordance with the requirements set out in Articles 13 and 14 of Regulation (EU) No 1257/2013. These emission allowances must be replaced by 1 March 2015. Special conditions for ship recycling facilities are to be laid down in accordance with the procedure laid down by the Rules;

The following subparagraph is added:

No D1-1049, 2014-12-19 published TAR 2014-12-22, i.e. 2014-20406

2.13. where an IPPC permit has been issued in accordance with Annex 2 to the 2002 IPPC Regulations for several installations or parts thereof operated by a single operator located outside the same location, when amending the IPPC permit in accordance with the procedure laid down in the Rules, the permit (or allowances) shall be issued taking into account the provisions of paragraph 10 of the Regulations. In this case, the details of the title sheet (cover) of the permit (or allowances) shall be drawn up in accordance with the requirements of Annex 5 (information) to the Rules;

The following subparagraph is added:

No D1-252, 2015-03-30 published TAR 2015-03-31, i.e. 2015-04852

Amendments to point.:

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

2.14. The rules shall not apply to installations which comply with the criteria of Annex 2 to the IPPC Regulations 2002 and for which an IPPC permit has been issued, but which, in accordance with Order No D1-528 of the Minister of Environment of the Republic of Lithuania of 15 July 2013, must hold a permit issued in accordance with the Rules for Issuing, Replacement and Revocation of Integrated Pollution Prevention and Control Authorisations as of 7 July 2015;

The following subparagraph is added:

No D1-252, 2015-03-30 published TAR 2015-03-31, i.e. 2015-04852

2.15. IPPC permits issued in accordance with the IPPC rules for 2002 shall be replaced by emission allowances in accordance with the following procedure:

2.15.1. an application for renewal of a permit shall be submitted and considered in accordance with the procedure laid down in the Rules;

2.15.2. during the coordination of the application, the AAA shall have the right to request, on a reasoned basis, to supplement the application with information missing to establish permit conditions in accordance with the requirements of the Rules;

2.15.3. in case of changes of more than 50 % of the permit data or less than 50 % of the changes, but the operator requests in writing, the authorisation must be changed in full. In such a case, the AAA shall have the right to request a supplement to the application on a reasoned basis;

2.15.4. the particulars of the title sheet (cover) of the renewal permit shall be drawn up in accordance with the requirements of Annex 5 (information) to the Rules, including the registration number of the changed IPPC permit, the name of the regional environmental protection department which issued it, the dates of issue, all updates and/or adjustments (if any) made;

2.15.5. the permit shall be assigned a number indicating the number of the replacement IPPC permit and the new number AAA in accordance with the procedure laid down by the AAA;

2.15.6.Repealed as of 07 January 2020.

Deletion of sub-paragraph:

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

2.15.7. by replacing an IPPC permit in accordance with the procedure laid down in subparagraphs 2.15.1-2.15.6 of this Order, all updates and/or adjustments of the IPPC permit previously carried out shall remain in force as an integral part of the permit, provided they have not been revoked by way of replacement of the IPPC permit with the permit or in accordance with the procedure laid down by other legal acts;

The following subparagraph is added:

No D1-252, 2015-03-30 published TAR 2015-03-31, i.e. 2015-04852

2.16. operators who did not hold a permit before the entry into force of this Order or held a permit without a special part of the permit entitled “Processing (recovery or disposal including preparation for recovery or disposal) and storage” (hereinafter referred to as the special part of the permit) or with a special part of the permit which had not previously been subject to the conditions set out in point 10 of the table in Appendix 2 of Annex 1 to the Regulations, as well as operators holding an IPPC permit issued in accordance with the criteria set out in Annex 2 to IPPC rules.

The following point is added:

No D1-754, 2015-10-23 published TAR 2015-10-26, i.e. 2015-16737

Amendments to point.:

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

2.17. operators operating combustion plants with a rated thermal input of 1 MW or more but less than 20 MW and falling within the scope of LAND 43-2013 of emission standards for combustion plants approved by Order No. D1-244 of the Minister of Environment of the Republic of Lithuania of 10 April 2013 “On the Approval of the Emissions from Combustion Plants LAND 43-2013” and a permit with a special part “Environmental Air Pollution Management”

2.17.1. the rated (nominal) thermal input of the combustion plant exceeds 5 MW – from 1 January 2024 with the permit conditions to come into force on 1 January 2025, established according to norms of emissions from medium combustion plants approved by Order No D1-778 of the Minister of Environment of the Republic of Lithuania of 18 September 2017 on the approval of norms of emissions from medium combustion plants (hereinafter: ‘the requirements for medium-sized combustion plants’)

2.17.2. when the rated thermal input of the combustion plant is 1 MW or more, but less than or equal to 5 MW – as of 1 January 2029, subject to permit conditions which will enter into force on 1 January 2030 and set in accordance with the requirements of the Average Combustion Plant Norms for existing combustion plants with such rated (nominal) thermal capacity;

Amendments to point.:

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

2.18.Repealed as of 07 January 2020.

Deletion of sub-paragraph:

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

The following subparagraph is added:

No D1-995, 2017-12-14 published TAR 2017-12-18, i.e. 2017-20260

2.19. Operators operating installations that meet the criteria of Appendix 1 to Annex 1 to the Rules and which have been issued a permit until and including 31 December 2021 shall exchange allowances by 1 January 2022 to receive the specific part of the permit ‘Flood Management’ by 1 January 2022. An application for the modification of an authorisation must be submitted to the AAA according to the time limits set in the Rules for the coordination of the application and the granting of the authorisation.

The following subparagraph is added:

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

2.20. The requirement set out in paragraph 41.1 and point 48 of the Rules to publish applications on the AAA website shall apply from 1 January 2021.

The following subparagraph is added:

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

3.I entrust:

3.1. The Department of Environmental Protection under the Ministry of Environment ensure compliance with the conditions specified in the permits issued in accordance with the requirements of the Rules on the grounds specified by the Law of the Republic of Lithuania on Environmental Protection and the Law on State Control of Environmental Protection of the Republic of Lithuania;

Amendments to point.:

No D1-536, 2018-06-19 published TAR 2018-06-20, i.e. 2018-10130

3.2. For the Environmental Protection Agency:

3.2.1. as of 1 July 2014, on the grounds specified Article 192 of the Law of the Republic of Lithuania on Environmental Protection and in accordance with the procedure laid down in the Rules, issue, review, amend and revoke permits;

3.2.2. as of 1 July 2014, on the grounds specified in Article 192 of the Law of the Republic of Lithuania on Environmental Protection and in accordance with the procedure laid down in the Rules, to amend IPPC permits issued in accordance with the criteria of Annex 2 to the IPPC Regulations 2002 and to cancel them;

3.2.3. publishing electronic versions of allowances issued or modified in accordance with the Rules during the period of validity of the permit and the cancellation information on the website of the Environmental Protection Agency;

The following subparagraph is added:

No D1-995, 2017-12-14 published TAR 2017-12-18, i.e. 2017-20260

3.2.4.collecting, systematising, storing and otherwise managing the data submitted by operators in accordance with the procedure laid down in the Rules of Procedure for each combustion plant (name of operator, address of the installation, date of commencement of operation of the installation, date of commencement of operation/exploitation or, if the exact date is not known, documents proving that the installation has started operations (commended in operation) before 20 December 2018, type of thermal power of the installation, type of fuel used for the purpose of collecting information of the Class of Activities of the Republic of Lithuania (According to the Statistical Class 226 of Activities and Statistics on the Statistical Classification) (Nominal) Heat Power of the Unit, Type of Use of Fuels Used for the Purpose of Purpose of Purpose of the Statistical Class of Activities of the Republic of Lithuania and the Statistical Class of Operations (Aclaration of Economic Activities) for the Purpose of the Purpose of the Purpose of Purpose of Purpose of Purpose of Purpose of Purpose of Use.

The following subparagraph is added:

No D1-995, 2017-12-14 published TAR 2017-12-18, i.e. 2017-20260

Amendments to point.:

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

3.2.5. if an installation referred to in Annex 1 to the Regulations (part of it, several installations or parts thereof) was issued an IPPC permit before 1 July 2014 in accordance with the IPPC Regulations 2002, free of charge (with the exception of the cases referred to in subparagraph 3.2.5.3 of this Order) to replace it with an emission permit complying with the requirements of the Regulations and containing the relevant special parts, in accordance with the following procedure:

3.2.5.1.by 1 July 2018, to approve and publish on the website of the Environmental Protection Agency a description of the procedure for preparing a draft permit, requesting and submitting additional data where they are lacking for drawing up permit conditions in accordance with the requirements of the Regulations, the procedure and time limits for familiarisation of operators to the draft permit and the form of confirmation of operators that the activities carried out are unchanged and conform to the environmental protection conditions of the installation (part of the installation, several installations or parts thereof) specified in the draft permit. The prepared draft procedure for obtaining conclusions of this subparagraph to the Ministry of Environment and interested entities;

3.2.5.2. initiating the replacement of allowances by notifying operators of the action plan for the replacement of IPPC allowances issued in accordance with the IPPC rules of 2002 prepared in accordance with the AAA and approved by 1 July 2018. Submit the draft action plan referred to in this point to the Ministry of Environment and stakeholders for information and publish the approved action plan on the website of the Environmental Protection Agency;

3.2.5.3. if the IPPC permit issued in accordance with Annex 2 of the IPPC Rules 2002 is renewed for any of the conditions of renewal of the permit specified in Article 192 of the Law of the Republic of Lithuania on Environmental Protection, by replacing it with the requirement of subparagraph 3.2.5 of this Order by replacing it with a pollution permit complying with the requirements of the Rules and containing the relevant special parts. In such a case, the AAA shall have the right to request additional data in accordance with paragraph 3.2.5.1 of this Order if they are missing for the permit conditions. When changing a permit in accordance with the procedure laid down in this subparagraph, the relevant requirements of the Rules for the replacement of a permit shall apply and state fees and charges of the established amount for the renewal of the permit must be paid;

The following subparagraph is added:

No D1-29, 2018-01-16 published TAR 2018-01-16, i.e. 2018-00674

3.2.6. in compliance with Resolution No. 937 of July 18th 2012 of the Government of the Republic of Lithuania on the Approval of the Profile of Licensing Basics, provide information to the License Information System about the issued or modified allowances, revocation of permits on the day of issuance, replacement or revocation of emission permits respectively;

The following subparagraph is added:

No D1-29, 2018-01-16 published TAR 2018-01-16, i.e. 2018-00674

3.2.7. by publishing the information referred to in point 90 of the Rules, specify the dates of receipt and acceptance of applications for, acceptance of allowances, issuance and replacement of allowances and decisions to adopt and/or not to accept applications;

The following subparagraph is added:

No D1-29, 2018-01-16 published TAR 2018-01-16, i.e. 2018-00674

Amendments to point.:

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

3.3. To advise the structural divisions of the Ministry of Environment, within their remit, on the application of the requirements laid down in the Rules;

3.4. I find that the requirement to carry out an environmental impact assessment or a screening for environmental impact assessment in accordance with the procedure laid down in the Law of the Republic of Lithuania on Environmental Impact Assessment of the Proposed Economic Activity shall not apply in accordance with the procedure laid down by the Law of the Republic of Lithuania on Environmental Impact Assessment of the Proposed Economic Activity, with a permit issued in accordance with Annex 2 of the IPPC Rules, with a pollution permit issued in accordance with the procedure laid down in point 3.2.5 of this Order, with the exception of the cases where it is planned to change the nature or functioning of the installation referred to in the Rules (part of the installation, several installations or parts thereof) or it is planned to extend it, where such a change or extension may have significant adverse effects on human health or the environment as specified in the Law of the Republic of Lithuania on Environmental Impact Assessment of the planned economic activity.

The following subparagraph is added:

No D1-29, 2018-01-16 published TAR 2018-01-16, i.e. 2018-00674

Minister of Environment						 Valentinas Mazuronis

APPROVED BY

Minister of Environment of the Republic of Lithuania Order No D1-259 of 6 March 2014

(Minister of Environment of the Republic of Lithuania Order No D1—425 of 16 July 2020

version).

rules ON the granting, UPDATING and revocation of pollution permits

CHAPTER I

GENERAL PROVISIONS

1. The rules on the granting, updating and revocation of pollution permits (hereinafter: ‘the Rules’) shall lay down the procedure for drafting, filing, publishing, examining and accepting an application for the issue or renewal of a permit referred to in Article 192 of the Law of the Republic of Lithuania on Environmental Protection, preparing a draft permit, issuing a permit, reconsidering the conditions of a permit, amending the permit and revoking the permit. The rules shall establish the measures for abstraction of water from surface water bodies and pollution reduction measures provided for in the Law of the Republic of Lithuania on Water, implementation of environmental air pollution management measures provided for in the Law of the Republic of Lithuania on Environmental Air Protection, waste prevention and management measures provided for in the Law of the Republic of Lithuania on Waste Management, climate change management measures provided for in the Republic of Lithuania Law on Financial Instruments for Climate Change Management, planning and implementation of other environmental protection measures.

2. The rules have been developed in accordance with Directive 2000/60/EC of the European Parliament and of the Council of 23 October 2000 establishing a framework for Community action in the field of water policy, Council Directive 91/271/EEC of 21 May 1991 concerning urban waste treatment (as last amended by Commission Directive 1998/15/EC, Council Directive 98/15/EC of 2006 February 2006 and Council Directive 2006/2006/EC of 15 February 2006 on recycling of certain hazardous substances, as last amended by Council Directive 2006/2006/EC of 2008 November 2006, and Council Directive 2006/2006/15/EC of 2006 November 11 on waste-related waste, as amended by Council Directive 2006/15/98/15 of 98 November 2006 and amended by Council Directive 2006/15/98/15 of Council Directive of 2006/15/98/15/2006 on waste from certain hazardous substances discharged into the Community, as amended by Council Directive 2006/15/98/15/2006 of Council Directive of 98/2006/98/15/98 of 2006/15/2006, as amended by Council Directive 2008/98/2006/98 of 98/2006/98, as amended by Council Directive 2006/15/98/98/15/98/15/2006/2006, as amended by Council Directive 2006/15/2006/98 of 98/98/2006, as amended by Council Directive 2006/98/15/98 of 2006/15/98/2006, as amended by Council Directive 2006/15/98/98/15/15/2006 of Council Directive of 15 February 2000 on waste water, as amended by Council Directive 2006/15/98/15/EC of 15 November 2006 on waste from the European Parliament and of the Council of 15 November 2006, as amended by Council Directive 2006/15/98/98/15/EC of 15/2006/15/EC, as amended by Council Directive 2006/15/98/15/2006/2006, as amended by Council Directive 2006/15/2006/98/15/EC of 2006/15/15/2006, as amended by Council Directive 2006/15/98/98 of 2006/15/15/2006, as amended by Council Directive 2006/15/98/EC of 98/98/98/98/15/EC, as amended by Council Directive 2006/15/2006/98 of 2008/98/15/EC of 98/98/2006/15/2006, as amended by Council Directive 2006/15/2006/2006 of 15 February and Council Directive 2006/2006/15/2006 of which the Council Directive of 98/2006/98/15/2006/EC as amended by Council Directive 2006/2006/15/EC of 2006/15/2006, as amended by Council Directive 98/2006/98/EC of 2006/15/2006, as well as amended by Council Directive 98/2006/98/15/EC of 2006/15/2006, as well as the Council Directive of 98/2006/2006/15/2006, as amended by Council Directive 2006/15/15/EC of 2006/15/EC of 2006/11/EC, as amended by Council Directive 98/2006/2006/EC of 2006/98/EC of 98/2006/EC, as amended by Council Directive 2006/15/2006/98/EC of 15 February 2000 on emissions of certain hazardous substances discharges into the Community, as amended by Council Directive 2006/15/98/EC of 98/98 and Council Directive of 2006/15/15/98 for which Council Directive of 2006/15/2006 and Council Directive of 98/2006/15/EEC of 2006/21/98/EC amended Council Directive 98/2006/15/EC of 15 February 2006 concerning pollution caused by certain hazardous substances discharged in the Community, as amended by Council Directive 98/15/EC of 98/98 and Council Directive 98/2006/98/15/EC of 15/2006/15/EC, laying down a framework for Community action in the field of water policy, establishing a framework for Community action in the field of water policy.

3. The rules must be followed by operators operating equipment as defined in point 4.3 of the Rules; The Environmental Protection Agency (hereinafter referred to as the “AAA”) issuing permits, reviewing their conditions, amending permits and withdrawing them, the National Public Health Centre under the Ministry of Health of the Republic of Lithuania (hereinafter referred to as the “NISC”), establishing the conditions for the management of odour and controlling compliance with these conditions, and the Department of Environmental Protection under the Ministry of Environment (hereinafter referred to as the AAD), which submits comments and/or proposals on applications for the granting or replacement of permits and controls compliance with the permit conditions. The rules shall not apply to research and experimental development (hereinafter referred to as ‘R & D’) or to testing of new products and processes, provided that the R & D activity or testing of new products and processes does not exceed 2 years, as well as for the activities referred to in Appendix 2 of Annex 1 to the Rules.

CHAPTER II

DEFINITIONS

4. The following definitions shall be used in the Rules:

4.1. operator means a natural person, a legal person or a branch thereof (including a foreign legal person and another organisation, as well as a branch thereof) that operates or controls an installation in whole or in part of an installation which owns or uses an installation, manages the installation by lease, use, trust or otherwise is responsible for controlling the operation and technical functioning of the installation;

4.2. emission or discharge limit value means the amount of emissions or releases expressed in certain parameters, concentrations and/or levels that may not be exceeded in one or more periods;

4.3.installation means a stationary technical unit in which one or more activities complying with the criteria listed in Annex 1 to the Regulations are carried out and any other directly related activity carried out at the same site, which may cause pollution and/or other consequences or effects on the environment;

4.4. abnormal (non-conformity) operating conditions means start-up, adjustment, braking of an installation as described in the operating document of the installation (technical regulation, etc.), as well as the presence and failure of leakage;

4.5. the concepts “existing combustion plant”, “new combustion plant”, “average combustion plant”, “nominal thermal input”, “operating hours” shall be understood as defined or consumed in the norms of emissions from medium combustion plants approved by Order No D1-778 of the Minister of Environment of the Republic of Lithuania of 18 September 2017 on the approval of norms for emissions from medium combustion plants (hereinafter – the norms for medium combustion plants).

5. Other concepts used in the Rules shall be within the meaning of the Civil Code of the Republic of Lithuania, the Law of the Republic of Lithuania on Environmental Protection, the Law of the Republic of Lithuania on Water, the Law of the Republic of Lithuania on Environmental Air Protection, the Law of the Republic of Lithuania on Waste Management, the Law of the Republic of Lithuania on Financial Instruments for Climate Change Management, the Law of the Republic of Lithuania on Environmental Monitoring and other legal acts.

CHAPTER III

GENERAL PRINCIPLES FOR GRANTING AUTHORISATIONS

6.An operator operating an installation (part of an installation, several installations or parts thereof) meeting one or more of the criteria set out in Annex 1 to the Regulations shall be required to have a permit.

7. A permit issued for the operation of an installation may contain one or more specific parts, taking into account which criteria of Annex 1 to the Regulations are met by the installation.

8. An operator operating an installation referred to in paragraph 6 of the Regulations meeting the criteria specified in Appendix 1 to Annex 1 to the Regulations must have a specific part of the permit for odour management.

Amendments to point.:

A note from the tar.Subparagraph 8 of the Rules for the Issuance, Replacement and Revocation of Pollution Permits shall enter into force on 1 January 2021.

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

9. The authorisation shall be issued, modified, reviewed, the conditions of the permit are specified, and the authorisation shall be revoked by the AAA.The permit shall be prepared or replaced on the basis of an application adopted by decision of the AAA.

10. A single permit may be granted to one installation, part thereof or several installations (or parts thereof) operated by the same operator and located in the same location (i.e. on a parcel, adjacent plots or segregated plots with the engineering infrastructure only).At the request of an operator, a single permit may be issued to several installations operated by the same operator (or parts thereof) meeting criteria 1.1-1.4 of Annex 1 to the Regulations and located in the territory of one municipality, where by requesting such a permit, the operator states in writing that it understands that, in the event of an infringement or violations specified in paragraph 11 ofArticle 192 of the Law of the Republic of Lithuania on Environmental Protection in one of the installations, the permit issued to several installations (or parts thereof) may be revoked in accordance with the procedure laid down in the Rules. Where several operators operate or manage an installation (part of it, several installations or parts thereof) on ownership, lease, loan for use, trust or other legal basis, a single permit may be issued (or resubmitted to include several operators, if previously issued to a single operator), specifying the details of the operators and the sharing of responsibility.

11. Where a permit is issued for a part of an installation or parts of several installations, the permit shall specify the name(s) of the entire installation or several installations, its/their design capacity, part of the installation (or parts of installations) for which the permit and its/their design capacity are issued.An installation may not be divided into parts with a view to carrying out an activity or operating part of an installation without a permit.

12. The operator must obtain a permit for operating a new installation (part of it, several installations or parts thereof) before the start of operation of the installation.

13. When planning a change or extension of an installation which results in permit conditions other than those laid down in the permit, the permit must be amended or revised prior to the change or extension of the installation.

14. When planning to put into service a new or modified installation, an application for or a renewal of a permit must be submitted to the AAA according to the time limits set in the Rules for the coordination of the application and the granting of a permit.

15. A permit shall be issued or renewed only upon selection of the proposed economic activity for environmental impact assessment (hereinafter: ‘selection’) and/or environmental impact assessment, where such procedures are mandatory under the Law of the Republic of Lithuania on Environmental Impact Assessment of the Proposed Economic Activity (hereinafter referred to as the “EIA Law”), and upon receipt of the conclusion of the selection of the competent authority that environmental impact assessment is not mandatory (hereinafter referred to as the screening conclusion) or a decision on the environmental impact under which the proposed economic activity complies with the requirements of legal acts (hereinafter: the EIA decision), unless the permit is renewed in the course of the selection of the said authority (hereinafter: ‘selection conclusion’) or a decision on environmental impact, under which the proposed economic activity complies with the requirements of legal acts (hereinafter: the EIA decision), unless the permit is renewed in the course of the selection of the competent authority on the grounds which is not mandatory (hereinafter referred to as the screening conclusion) or a decision on the environmental impact under which the proposed economic activity complies with the requirements of legal acts (hereinafter referred to as the EIA decision), unless the permit is renewed in the course of the selection of the competent authority on the grounds that the environmental impact is not mandatory (hereinafter referred to as the screening conclusion), or a decision on the environmental impact under which the proposed economic activity complies with the requirements of legal acts (hereinafter: the EIA decision), unless the permit is renewed in the course of the selection of the competent authority on the grounds that the environmental impact assessment is not mandatory (hereinafter referred to as the screening conclusion) or the proposed economic activity under which the proposed economic activity complies with the requirements of legal acts (hereinafter: the EIA decision), except where the permit is amended for the reasons not related to the use of the environmental impact or the proposed economic activity, or the proposed economic activity, where the proposed economic activity is to comply with the requirements of legal acts (hereinafter: the EIA decision), unless the permit is renewed in the course of the selection of the proposed economic activity for the reasons not related to the exploitation of the PS, or the proposed economic activity is to be subject to the proposed environmental impact assessment, or where the proposed economic activity is intended to comply with the requirements of legal acts (hereinafter: the EIA decision), except where the permit is amended for the reasons not related to the permit to be used in respect of the permit or the extension of the permit, or the proposed economic activity will be subject to compliance with the requirements of legal acts (hereinafter:

16. Where screening or environmental impact assessment procedures have been carried out for the proposed economic activity, the permit issued and/or amended must comply with the conditions set out in the EIA decision and the scope and characteristics of the economic activity indicated in the EIA decision and/or the screening conclusion, the envisaged measures to reduce and/or compensate for the significant negative impact on the environment, which must be implemented during the preparation for the activities, during the implementation and completion of the activities.

17.Where the procedures set out in the EIA Law are not mandatory and the operator is required to obtain or modify a specific part of the permit under the requirements of these Rules, namely ‘Wastewater Management and Discharge’ or ‘Management of Environmental Air Pollution’, the operator shall demonstrate to the AAA that:

17.1. pollutants discharged with waste water in the course of activities in the environment will not exceed the set environmental quality standard (limit value or other value).An impact assessment must be carried out on the acceptance when it is required in accordance with the requirements of the Waste Water Management Regulation approved by Order No. D1-236 of the Minister of Environment of 17 May 2006 on the approval of the Waste Water Management Regulation (hereinafter referred to as the “Sewage Management Regulation”);

17.2. the pollutants emitted to ambient air during activities, including during unusual (non-attractive) operating conditions, will not exceed the ambient air quality standard established for them, established in the list of contaminants whose quantity in ambient air is restricted in accordance with the criteria of the European Union and the List of Pollutants whose quantity in ambient air is restricted according to national criteria and in the limit values of ambient air pollution approved by the Minister of Environment of the Republic of Lithuania and the Minister of Health of the Republic of Lithuania by order of 30 October 2000 the quantity of pollutants whose quantity of air is restricted according to the criteria of the European list of pollutants, the quantity of which is restricted according to the criteria of the European list of pollutants, according to the criteria of the European list of pollutants, the quantity of which is restricted according to the criteria of the European list of pollutants, the amount of which is restricted according to the criteria of the European list of pollutants, according to the criteria of the List of Pollutants of the Republic of Lithuania and the Minister of Health of the Republic of Lithuania. Therefore, an assessment of the impact of emissions on ambient air must be carried out (calculations of dissemination of pollutants in ambient air) taking into account the requirements set forth by Order No D1-653 of the Minister of Environment of the Republic of Lithuania of 30 November 2007 on the Calculation Models of Pollution Dissemination of Pollution of Pollution of Pollution of Pollution of Pollution of Pollution of Pollution of Pollution, the Procedure for the Use of Environmental Air Pollution Data and the Procedure for the Use of Meteorological Data on the Impact of Economic Activities on ambient air (hereinafter: ‘the procedure for background ambient air pollution and use of meteorological data’).

18. A greenhouse gas emissions permit from installations shall be issued, renewed and revoked in accordance with the procedure laid down by Article 7 of the Law of the Republic of Lithuania on Financial Instruments for Climate Change Management and the Procedure for Granting and Trading in Greenhouse Gas Emission Allowances approved by Order No D1-231 of the Minister of Environment of the Republic of Lithuania of 29 April 2004 on the Approval of the Procedure for Allocation of Greenhouse Gas Emission Allowances and Trading therein.

19. Authorisation shall be granted for an indefinite period, except for the special part of the permit “Ship Recycling”, but subject to permit conditions, the AAA may impose conditions for a certain period of time after which the conditions of the permit are to be reviewed, modified or withdrawn in accordance with the procedure laid down by the Rules.

20. The Special Part ‘Ship Recycling’ of the permit shall be issued for a limited period as laid down in Article 14(1) of Regulation (EU) No 1257/2013. Upon the expiry of the period of validity of the special part of this permit, a new special part “Ship Recycling” must be issued in accordance with the procedure laid down by the Rules. In such a case, the requirement of point 15 of the Rules to carry out the procedures laid down in the EIA Law shall apply if the conditions of ship recycling activities change and this change corresponds to the cases specified in the EIA Law. In order to qualify for the new Special Part entitled ‘Ship Recycling’, an application shall be submitted in accordance with the procedure laid down in the Rules, taking into account the requirement laid down in paragraph 14 of the Regulations and all the data and documents required by the Rules for the issue of this special part of the authorisation.

21. Compliance with the principles laid down in Article 192 of the Law of the Republic of Lithuania on Environmental Protection must be ensured when issuing or amending permits, setting permit conditions and operating installations.

CHAPTER IV

REQUIREMENTS FOR THE PREPARATION OF AN APPLICATION FOR A PERMIT OR MODIFICATION OF AN AUTHORISATION

22. A model for the application for or modification of an authorisation is given in Annex 2 to the Rules. The application shall consist of the title sheet, the single part of the application, the specific part(s) of the application. Other documents shall be submitted in the cases and in accordance with the procedure laid down in the Rules.

23. Data and information (plans, schemes, extracts, extracts, etc.) from the territorial planning documents prepared in compliance with the Law of the Republic of Lithuania on Territorial Planning, the documents of information selection or environmental impact assessment documents drawn up in compliance with the Law on the EIA, from a technical or construction project, the report on the approval of the inventories of ambient air pollution and their emission inventories approved by an order of the Minister of Environmental Air Pollution and their emission inventories, approved by an order of the Minister of Environment on the Approval of Statement of Air Pollution from 27 June 2002, and the regulations on the Approval of the Environmental Sources of Environmental Pollution from the Environmental Sources and the Guidelines for the Provision of the Invention of Pollutants, approved by the Minister of Environment for Health, and the Regulations on the Approval of the Impact Assessment of Air Pollutants from June 27, and the Regulations on the Approval of the Environmental Pollution from the Sources of Environmental Pollution and Reports on the Assessment of Pollutants from them, approved by the Minister of Environment by the Order of the Approval of the Impact Assessment of Air Pollutants from 27 June 491 and the Regulations on the Approval of the Environmental Impact Assessment from the Sources of Environmental Pollutants and the Environmental Pollutant Assessment Reports, approved by the Minister of Environment by Order of the Environmental Impact Assessment Reports on the Impact Assessment of Air Pollutants and on the Approval of the Environmental Impact Assessment Reports from the Minister of Environment on 27 June.

24. The title page of the application shall include the name of the operator, the code of the legal person, the address of the registered office, the contact person’s data, the name and address of the object of the economic activity, the criteria (criterion) set out in Annex 1 to the Rules, which the installation fulfils and, if it meets, the criteria (criteria) of Appendix 1 to Annex 1 to the Rules.

25. The single part of the application shall specify:

25.1. in the descriptive section: information about the installation (part of it, several installations or parts thereof), the activities carried out and intended to be carried out by the installation:

25.1.1. brief descriptive information on all installations operated and/or planned to be operated by the same operator at that site (or several sites, if a permit is requested for several installations located in the territory of one municipality) and which are likely to cause emissions or discharges, specifying the technical parameters of the installations irrespective of whether the installations comply with paragraph 4.3 of the Rules;

25.1.2. the design capacity of a planned installation or installation in accordance with the criteria specified in Annex 1 to the Regulations, a detailed description of the activities carried out and planned in the installation or installations and the technologies used (including sources of emissions or discharges, emissions or releases, if not included in the specific parts of the application).For a new installation, an indication of the start of construction and the planned start of activity, for an existing installation the operation of which is planned to be changed or extended, and the intended start of operations following the replacement of a permit;

25.1.3. in the case of an application for a permit for operation of combustion plants, documentary evidence of their rated (nominal) thermal input, type (diesel engine, gas turbine, dual-fuel engine, other engine or other combustion plant), information on the annual number of operating hours (where the number of operating hours is declared in accordance with paragraph 36.5 of the Regulations); for existing medium combustion plants, where the exact date of commencement of their operation/operation is not known, documentary evidence shall be provided that the plant was put into operation (commated) before 20 December 2018;

That the equipment meets at least one of the criteria specified in Appendix 1 to Annex 1 to the Regulations; if yes, specify a specific criterion (criteria);

25.1.5. conditions of the site of operation of the installation (geographical conditions of the elements into which the pollution will be emitted or discharged according to individual emissions or discharges during the operation of the installation (mountain, valley, etc., open uninhabited area, etc.).The background level of ambient air pollution shall be the ambient air pollution level assessed in accordance with the procedure for the use of background ambient air and meteorological data;

25.1.6. measures and actions to prevent or, where this is not possible, to reduce emissions or releases from the installation; where the intensity of activities carried out in the installation and the associated environmental pollution vary significantly per year (or per day) on the basis of the technology, or no activity is carried out during a given period, information shall be provided on the periods of activity of different intensity;

25.1.7. waste prevention measures provided for or used in the installation (applicable to undertakings other than waste management);

25.1.8. planned use of water sources, water demand, waste water treatment methods. This information shall not be provided if it is included in the specific parts of the application ‘Management and discharge of waste water’ and/or ‘Water extraction from surface water bodies’;

25.1.9. information on unusual/non-conforming conditions of operation of the installation and the measures planned to reduce pollution in order not to exceed the environmental quality standards; information on the possible duration of such conditions, justifying that the specified duration is as short as possible (unless this information is provided in the specific parts of the application);

25.1.10. the number of the document permitting the construction and the date when it is mandatory to have it in accordance with the procedure laid down by legal acts, and a reference to it if the document has been publicly announced; where screening or environmental impact assessment procedures have been carried out, a reference to the EIA decision or to the conclusion of the selection, indicating the date and number of the EIA decision or screening conclusion;

25.1.11. where screening or environmental impact assessment procedures have been carried out, detailed information on the implementation or implementation of the conditions specified in the EIA decision prior to the commencement of activities and the measures indicated in the EIA decision and/or selection conclusion to reduce and/or compensate for significant adverse effects on the environment, which must be implemented prior to the commencement of the activity or during the operation (operation of the installation);

25.1.12. if public health impact assessment has been carried out in accordance with the Law of the Republic of Lithuania on Public Health, reference shall be made to the documents of public health impact assessment. This information shall be provided if the installation meets at least one of the criteria specified in Appendix 1 to Annex 1 to the Regulations;

25.2. in the tables in the General Part, lists of feedstocks and auxiliary substances, including chemicals and mixtures, fuels, their quantities, risk/hazard and safety/precautionary phrases; for combustion plants, the type/species of fuel according to the fuels specified in the Average Combustion Plant norms.

26. Where an operator wishes to obtain a permit for operation of several installations or parts thereof, the information referred to in points 25.1.6 to 25.1.9 and 25.2 of the Rules must be submitted in the application separately (in separate tables) for each installation subject to a permit.

27. Specific parts of the application:

27.1. waste water treatment and discharge;

27.2. ambient air pollution management;

27.3. climate change management;

27.4. waste treatment (recovery or disposal, including storage and preparation for recovery or disposal);

27.5. extraction of water from surface water bodies;

27.6. ship recycling;

27.7. fragrances management.

Amendments to point.:

A note from the tar. Subparagraph 27.7 of the Rules for Issuing, Replacement and Withdrawal of Pollution Permits shall enter into force on 1 January 2021.

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

28. The applicant shall prepare the single part of the application and the specific part(s) of the application under which the authorisation is to be issued or modified. The criteria for operating a specific part of the permit referred to in point 60 of the Rules are specified in Annex 1 to the Rules. Each specific part of the application shall be drawn up in accordance with the model set out in the relevant Appendix to Annex 2 to the Regulations. If the activities carried out or planned at the installation do not cover certain areas for which information is requested (e.g. waste is used, but not disposed of, etc.) or the activities carried out have no particular impact on the environmental components or human health (no pollutants, etc.), no tables relevant to the application shall be filled in or submitted, but their name shall be retained, and a justification shall be given on the table as to why the information or data are not provided.

29. The specific part ‘Management and discharge of waste water’ shall be completed and provided by operators planning to operate an installation in which an economic activity fulfils the criteria set out in paragraph 1 of Annex 1 to the Rules. This Part contains the following information and documents (Appendix 1 to Annex 2):

29.1. the type of waste water generated (or planned to be generated), a surface water body (receiver) to which waste water is intended to be discharged;

29.2. amount of waste water planned to be discharged and their contamination, including with priority substances;

29.3. documents of inventory of pollutants discharged to the environment and/or the impact on the receiver of waste water drawn up in accordance with the requirements of the Waste Water Management Regulation;

29.4. programme for the reduction of pollution of waters with priority substances when planning or pollution of waters with these substances, drawn up in accordance with the requirements of the Waste Water Management Regulation;

29.5. additional information on discharge of surface waste water specified in the Waste Water Treatment Regulation approved by Order No. D1-193 of the Minister of Environment of the Republic of Lithuania on the Approval of the Regulation on Surface Waste Management" of 2 April 2007 (hereinafter referred to as the "Surface Waste Water Treatment Regulation");

29.6. a detailed diagram of the waste water treatment plant, showing all the untreated, treated and treated waste water flow lines and the valves that control them, where one waste water line is connected to another (e.g. fence line) and the places where effluent can enter the environment (discharger, emergency discharge).The elements listed in the waste water treatment plants must be marked in the same way as in this scheme.

30. The specific part of the application entitled ‘Environmental Air Pollution Management’ shall be completed and submitted by operators planning to operate an installation in which the economic activity complies with the criteria set out in paragraph 2 of Annex 1 to the Rules. This Part provides information on (Appendix 2 to Annex 2):

30.1. the expected emissions to ambient air from all installations and the total amount of pollutants expected to be emitted from each installation;

30.2. stationary sources of pollution for each installation emitting pollutants into ambient air. For the operation of new combustion plants, new combustion plants merged according to the norms of the average combustion plants shall be considered as one stationary source of pollution (combustion plant).For the operation of existing combustion plants, a group of existing combustion plants (boilers) shall be considered as one stationary source of pollution (combustion plant), where the fuel is oxidised for the purpose of using the heat energy generated by the incineration process and the exhaust gases are discharged into ambient air through a single stack. Where a group of combustion plants (boilers) consists of one or more new combustion plants and one or more existing combustion plants and their exhaust gases are discharged into ambient air through a single chimney, the new combustion plant or a group of combustion plants and an existing combustion plant or group of combustion plants shall be considered as separate stationary sources of pollution (combustion plants);

30.3. estimated pollution into ambient air from each stationary emission source of each installation;

30.4. ambient air pollution treatment equipment operated (intended to be operated) at the installation and applied measures for the prevention of ambient air pollution;

30.5. pollution into ambient air of each installation under unusual (extraordinary) operating conditions;

30.6. this special part of the application shall contain the following documents:

30.6.1. justifying the calculation of pollutant emissions to ambient air;

30.6.2. assessment of the impact of emission pollutants on ambient air, including assessment of the effect on ambient air in the event of unusual (non-conventional) operating conditions;

30.6.3. substantiating that the average combustion plant was put into operation (commended in operation) before 20 December 2018, where it is necessary to submit, in accordance with paragraph 25.1.3 of the Rules, documentary evidence that the plant has been put into operation (commended in operation) before the said date;

30.6.4. a document justifying the rated (nominal) thermal input of the combustion plant as issued by the producer or a document issued by a competent authority in accordance with the procedure established by the Ministry of Energy of the Republic of Lithuania;

30.6.5. justifying that the conditions of the permit requested for the average combustion plant comply with the emission limit values set out in Chapter IV of the Average Combustion Plant Standards. This documentation shall be submitted by the operator who intends to make use of the exemption(s) of the limit values referred to in Chapter IV of the average combustion plants in the cases referred to in the relevant points of that Chapter, where the AAA may exempt the operator from compliance with the limit values set out in the norms for certain medium combustion plants and shall ensure, in accordance with the requirements of point 17.2 of the Regulations, that emissions to ambient air during the activity do not exceed their ambient air quality standard.

31. The specific part of the application ‘Climate Change Management’ shall be completed and submitted by operators planning to operate an installation in which economic activities comply with the criterion set out in paragraph 5 of Annex 1 to the Rules. This part contains the following information (Appendix 2 to Annex 3):

31.1. planned greenhouse gas emissions (hereinafter referred to as “GHG”);

31.2. sources of the installation from which emissions into the atmosphere and the date of submission of the monitoring plan are submitted;

31.3. this special part of the application shall contain the following documents:

31.3.1. A GHG monitoring plan drawn up in accordance with the procedure laid down by Order No D1-168 of the Minister of Environment of the Republic of Lithuania on the Approval of Procedure for Monitoring, Monitoring and Reporting of Greenhouse Gas Emission Allowances and Operators Participating in the European Union Greenhouse Gas Emission Allowance Trading Scheme approved by Order No D1-168 of the Minister of Environment of the Republic of Lithuania of 26 February 2015 on the Approval of Procedure for Monitoring, Monitoring and Reporting of Greenhouse Gas Emissions Emissions Trading Scheme;

31.3.2. the operator’s obligation, as set out in Appendix 9 of Annex 2 to the Rules, to report by 31 March each year and to surrender, by 30 April each calendar year, the amount of GHG allowances equal to the amount of carbon dioxide emitted in tonnes and/or carbon dioxide equivalent during the previous calendar year.

32. The specific part of the application ‘Waste treatment (recovery or disposal, including storage and preparation for recovery or disposal)’ shall be completed and submitted by operators planning to operate installations meeting the criteria specified in Part 3 of Annex 1 to the Regulations, except as specified in Appendix 2 to Annex 1 to the Rules. This section shall contain:

32.1. information according to paragraphs A and/or B of Appendix 4 to Annex 2 to the Regulations;

32.2. additional information specified in legislation applicable to specific waste streams or categories:

32.2.1. in the application for a permit to operate an installation, which will carry out processing of end-of-life vehicles – additional data according to the requirements set out in the Rules for the Management of End-of-East Vehicles approved by Order No. 710 of the Minister of Environment of the Republic of Lithuania of 24 December 2003 on the Approval of Management Rules for end-of-life vehicles;

32.2.2. the requirements set out in the application for an authorisation to operate an installation, which will carry out the treatment of electrical and electronic equipment and its waste – additional data on compliance with the requirements set out in the Rules for the Management of Electric and Electronic Equipment and its Waste approved by Order No. D1-481 of the Minister of Environment of the Republic of Lithuania of 10 September 2004 on the Approval of Rules for the Management of Electrical and Electronic Equipment and its Waste;

32.2.3. in the application for a permit to operate a facility, which will perform the activities of the treatment of construction waste – additional data on the implementation of the requirements set out in the Rules for the Management of Construction Waste approved by Order No. D1-637 of the Minister of Environment of the Republic of Lithuania of 29 December 2006 on the Approval of Construction Waste Management Rules;

32.2.4. in the application for a permit to operate a facility which will carry out composting activities of biodegradable waste – additional data on compliance with the environmental requirements for composting biodegradable waste approved by Order D1-57 of the Minister of Environment of the Republic of Lithuania of 25 January 2007 “On the Approval of Environmental Requirements for Biodegradable Composting of Waste”;

32.2.5. in the application for a permit to operate a facility, which will carry out procurement activities for base metal scrap and waste – additional data according to the rules for accounting and storage of base metal scrap and waste and requirements for the installation of purchasing sites of base metal scrap and waste approved by Order No 4-678 of the Minister of Economy of the Republic of Lithuania of 6 September 2010 “On the Approval of Rules for Registration and Storage of Base Metal Steel and Waste, and Requirements for the Establishment of Purchasing Places of Base Metals and Waste”;

32.2.6. the requirements set out in the order No 625 of the Minister of Environment of the Republic of Lithuania of 21 December 2001 on the approval of the Rules for the Management of Batteries and Accumulators and Waste batteries and accumulators set forth in the application for a permit to operate an installation that will carry out the activities of the management of batteries and accumulators and waste batteries and accumulators – additional data on the implementation of the rules for the management of battery and accumulators and waste batteries and accumulators;

32.2.7. in the application for a permit to operate a waste incineration plant or waste co-incineration plant – additional data on compliance with environmental requirements for the incineration of waste approved by Order No 699 of the Minister of Environment of 31 December 2002 on the approval of environmental requirements for the incineration of waste;

32.2.8. in the application for a permit to operate a facility, which will carry out the activity of waste disposal in landfills – additional data in accordance with the requirements laid down in the Rules for Installation, Operation, Closure and Maintenance of Waste Landfills approved by Order No. 444 of the Minister of Environment of the Republic of Lithuania of 18 October 2000 “On the Approval of Rules for the Installation, Operation, Closure and Maintenance of Waste Landfills”;

32.2.9. in an application for an authorisation to operate an installation in which operations of mixing hazardous waste with other hazardous waste or substances, including the dilution of hazardous substances, will be carried out – additional data in accordance with the conditions laid down in the Law of the Republic of Lithuania on Waste Management;

32.3. this special part of the application shall contain the following documents:

32.3.1. technical regulation of waste recovery or disposal prepared according to Annex 3 of the Waste Management Rules approved by Order No. 217 of the Minister of Environment of the Republic of Lithuania of 14 July 1999 on the Approval of Waste Management Rules (hereinafter referred to as “the Waste Management Rules”) (hereinafter referred to as the “technical regulation of waste recovery or disposal”);

32.3.2. the plan for terminating waste recovery or disposal activities, drawn up in accordance with the provisions of the Description of the Procedure for Drawing up, Coordination and Implementation of the Plan for Termination of Waste Recovery or Disposal, approved by Order No 469 of the Minister of Environment of the Republic of Lithuania of 25 September 2003 on the Approval of the Plan for Decommissioning, Coordinating and Implementation of the Plan for Termination of Waste Recovery or Disposal Activities (hereinafter referred to as the “plan of cessation of waste recovery or disposal activities”).

33. The specific part ‘Water extraction from surface water bodies’ shall be completed and provided by operators planning to operate an installation in which the economic activity is carried out in accordance with the criteria set out in paragraph 4 of Annex 1 to the Rules. This part contains the following information (Appendix 2 to Annex 5):

33.1. name and category, identification code and exact location of the surface water body from which the water is to be extracted;

33.2. characteristics of the water body and the amount of water planned to be extracted. The amount of water extracted shall be calculated by dividing the annual amount of water extracted or expected to be extracted by the number of days of extraction;

33.3. information about fixed water extraction equipment (technical documents, construction project, etc.) shall be provided with this special part of the application.

34. The specific part of the application entitled ‘Ship recycling’ shall be completed and submitted by operators planning to operate installations meeting the criteria set out in point 6 of Annex 1 to the Rules. This part contains the following information (Appendix 2 to Annex 6):

34.1. ship recycling method(s) used in the installation;

34.2. the intended type and size of the ship in the most recyclable facility;

34.3. maximum quantities of vessels intended for recycling are planned to be stored in the installation at the same time;

34.4. maximum quantities of vessels to be processed in the installation simultaneously;

34.5. the planned maximum annual ship recycling capacity of the installation;

34.6. maximum amount of waste generated by ship recycling is planned to be stored in the facility (applies to installations where the waste generated on site is kept for less than the temporary storage period laid down in the Law of the Republic of Lithuania on Waste Management and the facility does not carry out waste treatment activities);

34.7. planned further treatment of waste (applies to installations where the waste generated on site is kept shorter than the temporary storage period laid down in the Law of the Republic of Lithuania on Waste Management and the facility does not carry out activities related to waste treatment);

34.8. if the facility is planned to process ships of 500 GT or more, information shall be provided on the hazardous materials contained in the structure and/or equipment of the ship to be recycled, including those specified in Annexes I and/or II to Regulation (EU) No 1257/2013 and how they will be stored and handled (if these materials do not become waste).For installations where ships of less than 500 GT are to be recycled, this information shall be provided in the case of ships designed and/or equipment containing dangerous substances which are intended to be recycled (Annex 2, Appendix 6, Table 3);

34.9. the application for a permit to operate an installation that will carry out ship recycling activities shall include further additional data on how the requirements set out in Article 13(1) of Regulation (EU) No 1257/2013 will be met for ship recycling facilities;

34.10. this special part of the application shall contain the following documents:

34.10.1. an application for inclusion in the European List of Ship Recycling Facilities in a free format, provided that the facility complies with the requirements set out in Regulation (EU) No 1527/2013;

34.10.2. the plan of the ship recycling facility;

34.10.3. local plan for response to pollution incidents.

35. The specific part of the application entitled ‘Quick Management’ shall be completed and submitted by operators planning to operate an installation in which an economic activity fulfils at least one of the criteria set out in points 1 to 6 of Annex 1 to the Regulations, provided that the installation carries out the activities listed in Appendix 1 to Annex 1 to the Rules. This part contains the following information (Appendix 2 to Annex 7):

35.1. data on stationary odour sources – sources of odour, exhaust gas indices (flow rate, temperature, volume flow), duration of action of odor sources;

35.2. map of the location of the source of odour marked for each source, information on compliance of odour emitted by the installation with public health safety legislation and supporting documents (laboratory tests, calculations);

35.3. fragrances management (reduction) measures and their efficiency indicators. Where a public health impact assessment has been carried out in accordance with the Law of the Republic of Lithuania on Public Health, a reference to the documents of public health impact assessment shall be provided;

35.4. documents proving the effectiveness of odour reduction measures.

Amendments to point.:

A note from the tar. Subparagraph 35 of the Rules for the Issuance, Replacement and Revocation of Pollution Permits shall enter into force on 1 January 2021.

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

36. Where applicable, the application shall contain the following documents or copies certified by the operator:

36.1. map where the location of the installation is marked in relation to the location of protected areas and biotopes, the strips and zones of protection of surface water bodies, the location of water protection zones, the sources of pollution, the extraction of water from the site of surface water bodies;

36.2. the programme for environmental monitoring of economic entities drawn up in compliance with the Regulations of Environmental Monitoring of Economic Entities approved by Order No D1-546 of the Minister of Environment of the Republic of Lithuania of 16 September 2009 on the Approval of the Regulations of Environmental Monitoring of Economic Entities;

36.3. the completed declaration contained in Appendix 8 to Annex 2 to the Regulations;

36.4. the declaration given in Appendix 9 to Annex 2 to the Rules is completed. To be completed by an operator operating an installation in which the economic activity complies with the criterion set out in paragraph 5 of Annex 1 to the Regulations;

36.5. the Declaration concerning the number of operating hours in Appendix 10 to Annex 2 to the Regulations is completed. It is completed by an operator who plans to operate an average combustion plant with a maximum operating time of 500 or 1000 operating hours per year (using a rolling average of 3 or 5 years) and wishes to benefit from the exemption from the limit values set out in point 19 or point 20 of the Average Combination Plant Standards;

36.6. A copy of the extract from the register of the central data bank of immovable property (land and structures); where the activity is carried out in a territory managed by the right of lease and/or structures – copies of lease contracts (which must be registered in the public register);

36.7. document confirming payment of state fees and charges for the issuance or amendment of a permit;

36.8. other documents which the operator deems necessary.

CHAPTER V

SUBMISSION, COORDINATION AND PUBLICITY OF AN APPLICATION FOR OR MODIFICATION OF AN AUTHORISATION

37. An application for the granting or renewal of an authorisation together with the documents required by the Rules shall be submitted to the AAA by e-mail or other electronic means of communication, either directly or through the point of single contact specified in the Law of the Republic of Lithuania on Services, unless it is technically impossible to reproduce or read the information submitted in this way. In this case, the application shall be submitted in paper format (seeded or tied) and on a digital medium. The application shall be accompanied by an accompanying document. When submitting an application in electronic format, the accompanying document and the declarations referred to in the Rules must be signed with a qualified electronic signature or formed by electronic means of communication which ensure the integrity of the text, irreplaceability and identification of the applicant. The application digital file must be submitted in *.docx or *.odt format and *.pdf format, other documents – *.pdf, *.docx, *.xlsx, *.odt, *.ods, *.ods, *.tif formats (optional).Annexes to the application, the submission of which is laid down by other acts only in paper format, or documents which may be submitted only in originals, shall be submitted separately in the accompanying document, with a separate list of paper documents in the accompanying document.

38. The applicant may indicate which data he considers to be a commercial/industrial secret according to the requirements of Article 1.116 of the Civil Code of the Republic of Lithuania, which are not disclosed to the public or are not otherwise disclosed to third parties without the applicant’s written consent. Such information and personal data must be separated and presented in a separate folder when submitting an application by electronic means or on a digital medium; in the case of an application in paper format, in a separate envelope. When submitting an application for coordination of other institutions, such information and/or data as well as personal data, the AAA must inform these institutions that the provided data and/or information are considered a commercial secret and ensure that such information is separated by electronic means of communication and provided in a separate folder, and when submitting an application in paper format – such information is provided in a separate envelope. The data indicated by the operator as a commercial/industrial secret may not be made public or made available to third parties without the written consent of the operator. When submitting an application for coordination of other institutions, such information and/or data as well as personal data, the AAA must inform these institutions that the provided data and/or information are considered a commercial secret and ensure that such information is separated by electronic means of communication and provided in a separate folder, and when submitting an application in paper format – such information would be provided in a separate envelope. The information and data which must be made available to the public in accordance with the procedure laid down by laws and other legal acts may not be considered a commercial/industrial secret.

Personal data shall be processed for the purposes of identification and communication of individuals in accordance with Regulation (EU) 2016/2016 of the European Parliament and of the Council of 27 April 679 on the protection of natural persons with regard to the processing of personal data and on the free movement of such data and repealing Directive 95/46/EC (General Data Protection Regulation) and the Law of the Republic of Lithuania on Legal Protection of Personal Data. Documents containing personal data shall be processed and stored in the document plan approved by the AAA, according to the Law of the Republic of Lithuania on Documents and Archives and the Rules on Documents Management and Registration approved by Order No. V-118 of the Chief Archivist of Lithuania of July 4, 2011 “On the Approval of Documents Management and Accounting Rules”. Upon the expiry of the period of storage of documents, the documents containing personal data shall be destroyed, except for those which, in the cases established by laws or other legal acts regulating data storage, shall be transferred for storage in compliance with the Law of the Republic of Lithuania on Documents and Archives, its implementing and other legal acts regulating the storage of such documents.

39. If an application has been made for a permit with a specific part of the application ‘Recovery (recovery or disposal, including storage and preparation for recovery or disposal)’, the application has to be aligned with the AAD.

40. If an application has been made for an authorisation or a modification of an authorisation with the specific part ‘Quick management’, the application has to be coordinated with the NSC.

41. An application for or modification of an authorisation shall be evaluated and adapted in accordance with the following procedure:

41.1. The AAA shall register the application, publish it (without attachments) on the website within 3 working days and, no later than within 15 working days of receipt, assess whether the application has been submitted in accordance with the requirements of the Rules, whether the application contains all the data and/or documents duly executed and/or sufficient information and data, including information obtained in accordance with the procedures laid down in the EIA Law, if any, to determine permit conditions;

41.2.an application for a permit with a specific part of the application “Recovery (recovery or disposal, including storage and preparation for recovery or disposal)” shall be submitted by the AAA no later than 3 working days from the date of receipt of the application and the documents contained therein in an electronic version of the AAD. The ELD’s comments and/or proposals concerning the plan for the cessation of recovery or disposal activities and the technical regulation of waste recovery or disposal, and, where an application for a permit is submitted, also concerning the application, shall be submitted by the AAA not later than within 7 working days from the date of receipt of the application from the AAA. In the event of failure to reply within the set time limit, it shall be considered that there are no comments and/or proposals for the documents submitted by the APD. After reviewing the submitted documents, the AAD provides an opinion on whether:

41.2.1. on the basis of the information provided in the plan for cessation of waste recovery or disposal activities on the closure and management and post-closure measures of waste recovery or disposal facilities, the WPD will be able to verify the proper implementation of the measures provided for in the Plan for Termination of Waste Recovery or Disposal Activities in case of cessation of the operator’s recovery or disposal activities;

41.2.2. information on the storage of waste and technological process of waste recovery or disposal (hereinafter referred to as the “technological process”) provided in the technical regulation of waste recovery or disposal (hereinafter referred to as the “technological process”) will allow, in the course of state control of environmental protection, to assess the total and individual quantities of waste at different stages of the technological process and the types of waste management activities carried out; whether, taking into account the results of the operator’s environmental control, the planned changes in waste recovery or disposal activities are possible to ensure compliance with the permit conditions and the requirements of the legislation regulating waste management;

41.2.3. envisaged changes to permit conditions specified in the application for amendment of a permit, taking into account the results of the operator’s environmental protection control possible for ensuring compliance with permit conditions and the requirements of legal acts regulating waste management;

41.3. an application for a permit with a special part of the application “Quick Management” shall be submitted by the AAA to the NISSC within 3 working days from the date of receipt at the latest in accordance with the following procedure:

41.3.1. the application with the accompanying document NSC shall be submitted by e-mail or other electronic means of communication in one of the formats specified in paragraph 37 of the Rules, unless it is technically impossible to reproduce or read the information submitted in this way. The accompanying document shall specify that comments must be submitted to the AAA and to the applicant who submitted the application. If a valid permit has been published on the AAA website, the AAA shall provide an exact link to the website at the time of the NSC reconciliation application for a change of permit;

41.3.2. In performing the functions set out in the Law of the Republic of Lithuania on Public Health and other legal acts, the NISSC is required to, within 7 working days from the date of receipt of the application from the AAA, assess, within its remit, whether the application complies with the requirements of the legal acts regulating public health safety in the field of odour management and requirements for the special part of the permit “Quick Management” and reply to the AAA and the applicant. If the NSC has no comments or proposals for an application, it shall coordinate the application, together with the specific part of the permit entitled ‘Quick management’ completed by the AAA.

Amendments to point.:

A note from the tar. Subparagraph 41.3 of the Rules for Issuing, Replacement and Withdrawal of Pollution Permits shall enter into force on 1 January 2021.

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

42. The public concerned shall have the right to consult the application and submit proposals or comments on the application and the granting or modification of an authorisation for 10 working days from the date of publication of the information on the received application on the AAA website.

43. Where the AAA is not sufficient to determine the conditions of authorisation, the AAA shall have the right to request the operator to provide additional data and information, specifying the legal basis and the specific conditions of the permit, at the time of harmonisation of the application.

44. When deciding to accept an application or not to accept an application, the AAA shall evaluate the comments and/or proposals submitted by the public concerned.

45. The decision to accept an application for the AAA shall be taken in accordance with the following procedure:

45.1. if the application in the cases provided for in the Rules has not been submitted to other institutions – having established that the application for the granting or amendment of a permit complies with the requirements of the Rules, the application contains all duly executed data and/or documents, sufficient information and data to determine the permit conditions, there are no conditions for non-acceptance of the application specified in Article 192 of the Law of the Republic of Lithuania on Environmental Protection, the AAA shall take a decision to accept the application and send it to the applicant within 3 working days from the adoption of the decision;

45.2. If an application was submitted for coordination of the APD in the case set out in point 41.2 of the Rules, after having established that the application for the authorisation complies with the requirements of the Rules, the application contains all the data and/or documents duly executed, sufficient information and data to determine the permit conditions, the APD has no comments and/or proposals for the application and/or proposals for the application and (or) there is no proposal for the application for non-acceptable documents and the submitted documents for which the application cannot be accepted, or do not provide comments within the deadline set in point 41.2 of the Rules.

45.3. if an application has been submitted for coordination with the NVSC in the case set out in point 41.3 of the Rules, the AAA shall take a decision to accept the application, write it on an AAA form and send it to the applicant within 3 working days of the decision, having established that:

45.3.1. the application for the granting or amendment of a permit complies with the requirements of the Rules, the application contains all the data and/or documents duly executed, sufficient information and data to determine the permit conditions and there is no one of the conditions for non-acceptance of the application specified Article 192 of the Law of the Republic of Lithuania on Environmental Protection;

45.3.2. The NSC has no comments and/or proposals for the application and submitted documents for which the application cannot be accepted or comments which do not relate to the information specified in Appendix 7 to Annex 3 to the Rules and to the measures planned to monitor odour, and shall, in accordance with the procedure laid down in paragraph 41.3.2 of the Regulations, provide the conditions of authorisation for the management of odour; or

45.3.3. within the time limit set in paragraph 41.3.2 of the Rules, the NVSC shall not submit comments and/or proposals for the application and/or documents submitted.

Amendments to point.:

A note from the tar. Subparagraph 45.3 of the Rules for Issuing, Replacement and Withdrawal of Pollution Permits shall enter into force on 1 January 2021.

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

46. The decision not to accept an application for an AAA shall be taken in accordance with the following procedure in the cases referred to in Article 192 of the Law of the Republic of Lithuania on Environmental Protection:

46.1. if the application in the cases provided for in the Rules has not been submitted to other institutions – having established that the application for the granting or renewal of a permit does not comply with the requirements of the Rules, the application does not contain all or inadequately executed data and/or documents, incorrect information has been provided in the application and/or other documents used to determine the conditions of the pollution permit, the AAA shall take a decision to reject the information or provide the information to the applicant, specify the information and data necessary to establish the permit conditions, or set another condition set out in Article 192 of the Law of the Republic of Lithuania on Environmental Protection, the AAA shall take a decision to reject the application or submit the information to the applicant at a later date, and shall provide the applicant with comments, which shall not be followed by the submission of comments. The application shall not be returned if it has been submitted by e-mail or other electronic means of communication;

46.2. If an application was submitted for coordination of the APD in the case set out in point 41.2 of the Rules – after having established that the application for the granting or amendment of a permit does not comply with the requirements of the Rules, the application does not contain all or inadequately executed data and/or documents, the APD shall provide comments and/or proposals for the application and/or the documents submitted in it, and the AA is not able to accept the application in it, which is not possible to accept the application, if the data required to establish the permit conditions, or the other decision to submit the application is not allowed to be adopted in Article 192 of the Law of the Republic of Lithuania. The application shall not be returned if it has been submitted by e-mail or other electronic means of communication;

46.3. If an application has been submitted for coordination by the NVSC in the case set out in point 41.3 of the Rules and the NSC has submitted comments or suggestions, the AAA shall decide not to accept the application which is made on the AAA form, attach comments by the NSC and send it to the applicant within 3 working days of the adoption of the decision, indicating to the applicant the obligation to revise the application and/or the documents contained therein – to submit missing documents or information or provide explanations together with the return of the application, unless the application was submitted by e-mail or other electronic means of communication, establishing that:

46.3.1. the application for the granting or amendment of a permit does not comply with the requirements of the Rules, the application contains incomplete or incorrectly executed data and/or documents, insufficient information and data to establish permit conditions or at least one of the conditions for non-acceptance specified in Article 192 of the Law of the Republic of Lithuania on Environmental Protection;

46.3.2. The NSC has submitted comments and/or proposals for the application and submitted documents for which the application cannot be accepted;

Amendments to point.:

A note from the tar. Subparagraph 46.3 of the Rules for Issuing, Replacement and Withdrawal of Pollution Permits shall enter into force on 1 January 2021.

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

46.4. if an application for the amendment of a permit is submitted because the case of replacement of a permit referred to in Article 192 (9)( 4) or (5) of the Law of the Republic of Lithuania on Environmental Protection has been established, the decision not to accept the application shall specify a time limit of 20 working days for submission of the adjusted application and it shall be indicated that in the event of failure to submit the revised application within the said time limit, the procedure of revocation of the permit will be initiated on the grounds specified in subparagraph 4 of paragraph11 of Article 192 of the Law of the Republic of Lithuania on Environmental Protection.

47. If a decision has been taken not to accept an application, following an application revised in accordance with the observations referred to in this Decision, the application shall be examined, coordinated and a decision whether or not to accept the application shall be adopted in accordance with the procedure laid down in paragraphs 39 to 49 of the Rules.

48. A decision to accept an application or, on the grounds specified in Article 192 of the Law of the Republic of Lithuania on Environmental Protection, to refuse the application shall be adopted not later than within 15 working days from the receipt of the application. The decision to accept the application and the accepted application shall be published on the AAA website not later than within 3 working days from the date of adoption of the decision.

49. If, in the cases specified in the Law of the Republic of Lithuania on Waste Management, an operator must have a suretyship insurance contract or a bank guarantee (hereinafter referred to as “financial security”), AAA shall, in accordance with the procedure laid down in the description of the plan for the termination, coordination and coordination of waste recovery or disposal activities, indicate to the operator about the decision on the coordination of the plan for the termination of waste recovery or disposal activities that taking into account the requirements set forth in Article 11 of the Law of the Republic of Lithuania on Waste Management, a decision to accept an application to obtain or amend the permit will be taken when all the requirements set out in Article 41 of the Law on Waste Management will be submitted by the operator. Once financial security has been provided by the operator, the AAA shall take a decision to accept the application no later than 4 working days from the date on which the financial collateral was provided to the AAA, unless it is established in accordance with the procedure laid down in point 41 of the Rules that the application cannot be accepted for other reasons and shall send the applicant a statement of acceptance within 5 working days of the adoption of the decision.

50. During the examination of the revised application, no comments may be made concerning the compliance of the application with the requirements of the Rules, the information and/or data required which the AAA did not provide and/or requested when examining the initial application, but could have requested it. This provision shall not apply if the AAA provides a reasoned justification for not submitting certain observations to the original application, or if other particulars or information have changed in the revised application, or other particulars or information have arisen which cannot be accepted in accordance with the procedure laid down in the Rules, although they have not been the subject of comments in the course of the examination of the application submitted for the first time.

51. Before deciding to accept an application or not, the AAA shall have the right, on its own initiative or at the request of the operator, to have an oral discussion with the operator. The results of discussions on comments shall be recorded in the minutes of the meeting and submitted to the operator together with the decision to accept the application or not to accept the application.

CHAPTER VI

REQUIREMENTS FOR THE PREPARATION OF AN AUTHORISATION OR ITS AMENDMENT

52. The authorisation shall be issued by the AAA in accordance with the model in Annex 3 to the Regulations.

53. The data provided in the operator’s application, documents submitted in the application, legal acts setting out environmental and, where appropriate, public health care requirements for the activities carried out in the installation (where such requirements have been established), EIA reports, EIA decision or information selection and selection findings (if the procedures laid down in the Law on EIA) are used shall be used to prepare or amend the permit.

54. When preparing the permit, the AAA must discuss with the operator any disputed issues, including the choice of method(s) for calculating emissions or releases, the choice of methods for measuring emissions or releases (concentration), and the evaluation of data.

55. The authorisation shall consist of:

55.1. the title sheet;

55.2. general part;

55.3. the special part(s) setting out the permit conditions;

55.4. additives to the authorisation:

55.4.1. An application for authorisation or amendment of an authorisation accepted by the AAA;

55.4.2. A monitoring plan for GHGs;

55.4.3. the programme for environmental monitoring of an economic entity;

55.4.4. technical regulation of waste recovery or disposal and the plan for terminating waste recovery or disposal activities;

55.4.5. a detailed diagram of the waste water treatment plant, showing all the untreated, treated and treated waste water flow lines and the valves that control them, where one waste water line is connected to another (e.g. fence line) and the places where effluent can enter the environment (discharger, emergency discharge);

55.4.6. in the case of a modified authorisation – the decision (decisions) of the AAA regarding the modification of the permit; where the conditions of the permit are specified, the AAA decision(s) concerning the adjustment of permit conditions referred to in paragraph 80.3 of the Rules;

55.4.7. list of additives to the authorisation (Appendix 9 to Annex 3) which shall be amended by amendments to the Annexes or by the AAA following a decision to amend or amend the authorisation.

56. Special parts of the permit shall be as follows:

56.1. Waste water treatment and discharge;

56.2. Ambient air pollution management;

56.3. Climate change management;

56.4. Treatment of waste (recovery or disposal, including storage and preparation for recovery or disposal);

56.5. Extraction of water from surface water bodies;

56.6. Ship recycling;

56.7. Odor management (issued when any other special part of the authorisation referred to in paragraphs 56.1 to 56.6 of the Rules is required).

Amendments to point.:

A note from the tar. Subparagraph 56.7 of the Rules for Issuing, Replacement and Withdrawal of Pollution Permits shall enter into force on 1 January 2021.

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

57. The following shall be considered under permit conditions:

57.1. data and/or information provided in the special part(s) of the authorisation;

57.2. the programme for environmental monitoring of the economic entity;

57.3.‘AAA’ means a decision (or decisions) to adjust the permit conditions referred to in paragraph 80.3 of the Rules;

57.4. data and/or information provided in the technical regulation of waste recovery or disposal:

57.4.1. the location of the waste, its area, storage conditions;

57.4.2. a description of the technological process of recovery or disposal of waste containing data on waste which is used and/or disposed of, the main stages of the technological process, the materials used in the course thereof, the resulting waste, waste water and the discharge of pollutants into ambient air;

57.5. a detailed diagram of the waste water treatment plant, showing all the untreated, treated and treated waste water flow lines and the valves that control them, where one waste water line is connected to another (e.g. fence line) and the places where effluent can enter the environment (discharger, emergency discharge).

58. The title page of the authorisation shall contain the name of the operator, the address of the registered office, the name and address of the object of the economic activity, the special parts constituting the authorisation, the dates of the granting and modification of the authorisation and any other information in the model of the title sheet of the authorisation provided for in Annex 3 to the Rules.

59. The general part of the authorisation shall specify:

59.1. details of the installation (part of the installation, several installations or parts thereof):

59.1.1. name of the object of economic activity (installation or installations), its design capacity and address;

59.1.2. if the installation (part of it, several installations or parts thereof) is operated or operated by several undertakings on ownership, lease, loan for use, trust or other legal basis, the specific liability of each undertaking (operator) in accordance with the declaration submitted;

59.2. information on economic activities carried out in each installation (part of an installation, several installations or parts thereof):

59.2.1. activities carried out in the installation (part of the installation, several installations or parts thereof) and criteria for the permit of the installation;

59.2.2. a brief description of the technologies and activities used in the installation (part of it, several installations or parts thereof);

59.2.3. when the permit is issued with the specific part “Climate Change Management” – which activity or activities include economic activity emitting GHG.

60. The permit conditions in one or more specific parts of the permit shall be determined in accordance with the criteria set out in Annex 1 to the Rules for an installation which fulfils which an installation is subject to a permit:

60.1. the special part of the permit “Management and discharge of waste water” (Appendix 1 to Annex 3) shall specify:

60.1.1. a body of water (receiver) into which discharge is permitted and the exact place of discharge;

60.1.2. the amount of waste water permitted to be discharged and their pollution (normatives), including with priority substances;

60.1.3. other conditions for waste water management and discharge shall be established in accordance with the requirements of legal acts regulating environmental protection;

60.2. the special part of the permit entitled ‘Management of ambient air pollution’ (Appendix 2 to Annex 3) shall specify:

60.2.1. a general list of the pollutants to be emitted into ambient air, the amount of each pollutant and their total amount;

60.2.2. permitted pollution into ambient air from each stationary source of pollution;

60.2.3. regulation of the operation of the installation through unusual (non-conformity) operating conditions notified by the operator in accordance with paragraph 25.1.9 of the Rules and regulation of the operation of the installation, taking into account the information provided in the application pursuant to paragraph 25.1.6 of the Rules about the periods of activity of different intensity;

60.2.4. other conditions where it is necessary to reduce ambient air pollution or improve ambient air quality;

60.3. in the special edition, ‘Climate Change Management’ (Appendix 3 to Annex 3), the following shall be specified:

60.3.1. sources of greenhouse gas emissions into the atmosphere and their name;

60.3.2. The date of submission and approval of the monitoring and monitoring plan for greenhouse gases;

60.4. the special part of the permit entitled ‘Recovery (recovery or disposal, including storage and preparation for recovery or disposal) of waste’ shall specify:

60.4.1. information according to paragraphs A and/or B of Appendix 4 to Annex 3 to the Regulations;

60.4.2. the obligation to have effective financial security, if this requirement is applied in compliance with the Law of the Republic of Lithuania on Waste Management, and other conditions for the treatment of waste, including storage and preparation for recovery or disposal, shall be established in accordance with the requirements of legal acts regulating environmental protection;

60.5. the special part of the permit entitled ‘Water extraction from surface water bodies’ (Appendix 5 to Annex 3) shall specify:

60.5.1. name and category of surface water body from which abstraction is permitted, identification code and exact location of water extraction;

60.5.2. allow to extract water;

60.5.3. other conditions for extraction of water from surface water bodies shall be established according to the requirements of legal acts regulating environmental protection;

60.6. the special part of the authorisation “Ship recycling” shall specify:

60.6.1. the ship recycling method(s) permitted in the installation;

60.6.2. the maximum type and size of the ship to be recycled shall be permitted in the installation;

60.6.3. maximum quantity of vessels intended for recycling permitted in the installation;

60.6.4. maximum quantity of vessels to be processed simultaneously in the installation;

60.6.5. maximum annual ship recycling capacity of the installation;

60.6.6. maximum quantity of waste generated during ship recycling to be stored simultaneously in the facility (applies to installations where the waste generated on site is kept shorter than the temporary storage period laid down in the Law of the Republic of Lithuania on Waste Management and does not carry out activities related to the treatment of waste);

60.6.7. further treatment of waste (applies to installations where waste generated by ship recycling is kept on the site where it is produced for a period shorter than the temporary storage period laid down in the Law of the Republic of Lithuania on Waste Management and does not carry out activities related to the treatment of waste);

60.6.8. restrictions and conditions of operation of the facility, including storage and/or management of hazardous waste;

60.6.9. storage and handling of hazardous substances, including those specified in Annexes I and/or II to Regulation (EU) No 1257/2013 (if they do not become waste after recycling);

60.6.10. operating restrictions and conditions of the installation with regard to the storage and handling of hazardous materials;

60.6.11. installation operating restrictions and/or other conditions for ship recycling in accordance with the requirements of legal acts regulating environmental protection;

60.6.12. the period of validity of this part of the special authorisation;

60.7. the special part of the permit entitled ‘Quality management’ shall specify:

60.7.1. sources of odour and maximum permitted odour emission index;

60.7.2. odor management/reducing measures (e.g. proper installation of tanks, covering or closing of tanks, collection and treatment of vapours generated by filling tanks, etc.);

Amendments to point.:

A note from the tar. Subparagraph 60.7 of the Rules for Issuing, Replacement and Withdrawal of Pollution Permits shall enter into force on 1 January 2021.

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

60.8. in the tables “Other conditions” of the specific parts of the authorisation, the requirements according to the information in the general part of the application required by paragraphs 25.1.7, 25.1.8 and 25.1.9 of the Rules, and other conditions necessary to ensure compliance with the requirements of the legislation specified in subparagraphs 29.5, 32.2.1 to 32.2.9 of the Rules shall be specified.

61. Each specific part of the permit shall be drawn up in accordance with the model set out in the relevant Appendix to Annex 3 to the Regulations. If the activities carried out or planned at the installation do not cover certain areas identified in the examples of the specific parts of the permit (e.g. waste is allowed to be used, but not disposed of, etc.) or the activity or authorised installation is authorised, the environmental components or human health are not affected (e.g. no specific pollutants, etc.), the relevant table of the specific parts of the permit shall not be filled in, but their serial number shall be retained, the name of the table shall be kept and a justification for not completing the table shall be entered on the table.

62. Where it is necessary to ensure that emissions or discharges of a pollutant from stationary sources of pollution from an installation spread in the environment do not exceed the environmental quality standard laid down in the legal act for that pollutant, the AAA shall, having assessed the data and documentation submitted in the application and the conditions of the site of operation of the installation, establish in the permit a more stringent limit value for the discharge or discharge of the pollutant than laid down in the legal acts regulating the pursuit of certain economic activities.

CHAPTER VII

AUTHORISATION, RECONSIDERATION OF PERMIT CONDITIONS AND PERMIT CHANGE

63. The decision to issue or modify the authorisation and the authorisation shall be made in electronic format (except in cases where there is no technical possibility to do so) and must be signed by a qualified electronic signature or formed by electronic means of communication which ensure the integrity of the text, its irreplaceability and the identification of the person making the decision. If the authorisation is issued in paper format, the person responsible for the AAA shall be signed on the title page of the permit.

64. A permit shall be issued or replaced or refused by a decision of the AAA in accordance with the conditions set out in Article 192 of the Law of the Republic of Lithuania on Environmental Protection. The decision to grant or vary an authorisation or not to grant or maintain an authorisation shall be made out on an AAA form. An authorisation or amendment thereto shall be drawn up on the basis of an accepted application.

65. An authorisation shall be issued, replaced or refused on a reasoned basis not later than within 20 working days from the date of acceptance of the application in the AAA. For objective reasons, the AAA may, by a reasoned decision and in compliance with the Law of the Republic of Lithuania on Public Administration, extend the time limit for the adoption of the decision for a period not exceeding 10 working days. The operator shall be notified in writing of the extension of the time limit for the granting or modification of an authorisation and shall state the reasons for the extension.

66. The authorisation shall be given a registration number; the registration number shall not be changed for the modified authorisation. The decision to issue or renew a permit must state that the operator must notify the AAA and the ELD in writing 10 working days in advance of the commencement of operation of a new or modified (extended) installation before the operation of a new or modified (extended) installation is put into operation.

67. The permit and documents related to its issuance or replacement shall be stored according to the procedure established in the Law of the Republic of Lithuania on Documents and Archives of the AAA.

68. A permit must be replaced in the presence of at least one of the conditions of replacement of a permit specified in Article 192of the Law of the Republic of Lithuania on Environmental Protection.

69. The operator shall submit an application for an AAA to change the authorisation, together with documentary evidence of any changes in data. When a permit is modified, the entire general part of the special part in which the data have changed shall be replaced if the particulars indicated therein have changed and a new title sheet (cover) of the permit is drawn up. Where only the additives referred to in points 55.4.2 to 55.4..4 of the Rules are amended, the application shall not be submitted without changing the conditions of the authorisation. These documents shall be amended in accordance with the requirements set forth for the drafting and/or amendment of these documents.

70. The decision of the AAA to change the authorisation shall be taken in accordance with the procedure and within the time limits laid down in the Rules for the issue of a new authorisation.

71. The operator must immediately notify the AAA of any planned changes or extensions in the nature or functioning of the installation that may have an impact on the environment (in the case of combustion plants, compliance with emission limit values); any planned changes to the waste treatment (recovery or disposal, including storage and preparation for recovery or disposal) carried out in the waste management facility. In the notification, the operator shall include information on the basis of environmental impact assessment documents or information for selection, where such procedures have been carried out, or details of planned changes and their impact on the environment.

72. In order to ensure compliance with the principles set out in point 21 of the Rules, to ensure that the installation is operated in accordance with the permit conditions and the permit conditions comply with the requirements of the legislation, the AAA must review the permit conditions in the following cases:

72.1. where the results of environmental monitoring or data on the quality of the environment otherwise obtained indicate that the pollution caused by the installation is permitted, but has a significant negative impact on the environment;

72.2. upon entry into force of new or changed requirements of directly applicable legal acts of the European Union of environmental protection, laws of the Republic of Lithuania regulating environmental protection and their implementing legal acts, normative standards of environmental protection, environmental protection standards and requirements of environmental quality norms, which establish permit conditions;

72.3. The adoption by the AAA of an inventory report in accordance with the rules for inventory and reporting of emissions from and sources of ambient air pollution; this provision shall not apply to the review of permit conditions for combustion plants;

72.4. at the request of the operator, where the authorisation lays down conditions for a certain period of time;

72.5. upon receiving the information specified in paragraph 71 of the Rules;

72.6. when the APD provides information on identified violations of permit conditions;

72.7. at the request of the operator, if no exceedance of the limit concentration of the hazardous substances referred to in Annex 2 to the Waste Water Management Regulation has been detected in discharges of at least 5 consecutive laboratory tests, at least one of them, in the case of discharges of at least one of them (if the hazardous substance is not listed in Annex 2 to the Waste Water Management Regulation or there is no limit for it);

72.8. where the authorisation lays down limit values for the hazardous substances specified in Annex 1 and/or Annex 2(A) and/or part B list B1 of the Waste Water Management Regulation and the conditions for discharge of such substances in waste water;

72.9. where the NSC, in compliance with the requirements of Article 192 (9)( 3) of the Law of the Republic of Lithuania on Environmental Protection, determines that, in the event of a change or entry into force of new legislation regulating public health care, stricter or additional conditions for the management of odour must be laid down in the permit or other odour reduction measures have to be applied and provide specific information for the AAA decision on the content of the AAA decision (i.e. whether the permit needs to be modified, the name of the installation, the applicable conditions to be amended and the reasons for the decision);

Amendments to point.:

A note from the tar. Subparagraph 72.9 of the Rules for Issuing, Replacement and Withdrawal of Pollution Permits shall enter into force on 1 January 2021.

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

72.10. where the NSC, in compliance with the requirements of Article 192 ( 9)(4) of the Law of the Republic of Lithuania on Environmental Protection, establishes 2 times a year that a natural or legal person has violated the permit conditions related to the management of odour and submits a decision that prevention of such violations needs to be tightened or additional permit conditions specifying the conditions to be changed and how they must be changed and provide justification for the AAA decision;

Amendments to point.:

A note from the tar. Subparagraph 72.10 of the Rules for Issuing, Replacement and Withdrawal of Pollution Permits shall enter into force on 1 January 2021.

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

72.11. having received from other institutions information about changes in the installation and/or activities carried out therein and violations of legal acts regulating environmental protection;

72.12. upon informing, in accordance with the procedure laid down by the General Regulations of Municipal Environmental Monitoring, approved by Order No D1-436 of the Minister of Environment of the Republic of Lithuania of 16 August 2004 “On the Approval of General Provisions of Municipal Environmental Monitoring”, the municipality or economic entity’s environmental monitoring data indicate that the pollution caused by an economic entity worsens the state of the environment and upon suggesting which and how conditions should be revised.

73. Conditions of authorisation In the case referred to in paragraph 72.8 of the Rules, the review shall take place at least every four years from the date of the first authorisation granted or modified pursuant to the Regulations. The conditions of the permit shall be reviewed in accordance with the requirements laid down in the Waste Water Management Regulation.

74. When reviewing permit conditions, the RAB shall use the results of the ELD inspections, the data of the environmental monitoring/monitoring carried out by an operator, and/or other information obtained in accordance with the procedure laid down in the Rules. Where information is insufficient to review and evaluate permit conditions, the AAA shall have the right to request information from the operator, including missing results of monitoring of emissions or releases and other data.The operator must provide the information referred to in this point to the AAA no later than 10 working days from the date of receipt of the request, if available, or explain in writing why it is unable to provide it.

75. When the permit conditions are reviewed in accordance with point 72.3 of the Rules, the following criteria shall be considered for the modification of the permit or for the revision of the permit conditions:

75.1. when activities are subject to the requirements of the Law on the EIA and the inventory report shall specify:

75.1.1. the permit does not specify a new source of pollution, a new pollutant in the source of pollution indicated in the permit, the concentration (maximum one-off amount) and/or the annual amount of the pollutant from the source of pollution indicated in the permit exceeds 25 per cent specified in the permit conditions – the permit shall be replaced according to the procedure laid down in the Rules following the procedures set forth in paragraph 6 of Article 3 of the EIA Law;

75.1.2. the concentration (maximum one-off amount) and/or the annual amount of the pollutant from the emission source indicated in the permit is not more than 25 per cent higher than specified in the permit conditions and the operator proves in accordance with the requirements of paragraph 17.2 of the Rules that the emissions to ambient air during the activity do not exceed the ambient air quality standard set for them or the concentration (maximum one-off amount) of the pollutant from the emission source indicated in the permit and/or the annual quantity is lower than the permit conditions. If the operator so requests, the conditions of the permit shall be adjusted in accordance with the procedure laid down in point 80 of the Rules on the basis of the inventory report;

75.2. when activities are not subject to the requirements of the Law on the EIA and the inventory report shall specify:

75.2.1. a new source of pollution not specified in the permit, new pollutant in the source of pollution indicated in the permit, emission concentration (maximum one-off amount) and/or annual amount of a pollutant from the source indicated in the permit shall be adjusted in accordance with the procedure laid down in paragraph 80 of the Rules, and/or the annual amount exceeds 25 % of the permit conditions:

75.2.2. the concentration (maximum one-off amount) and/or the annual amount of the pollutant from the emission source indicated in the permit is not more than 25 per cent higher than specified in the permit conditions and the operator proves in accordance with the requirements of paragraph 17.2 of the Rules that the emissions to ambient air during the activity do not exceed the ambient air quality standard set for them or the concentration (maximum one-off amount) of the pollutant from the emission source indicated in the permit and/or the annual quantity is lower than the permit conditions. If the operator so requests, the conditions of the permit shall be adjusted in accordance with the procedure laid down in point 80 of the Rules on the basis of the inventory report.

76. When the permit conditions are reviewed in accordance with point 72.5 of the Rules, after the operator has provided information on any planned changes to the activities carried out in the installation, the decision on the obligation to change the permit shall be taken on the basis of an assessment of the extent and nature of the proposed changes and their compliance with the provisions of the EIA Law. If it is established that the planned amendments comply with the requirements of Article 3(6) of the EIA Law, the permit shall be amended in accordance with the procedure laid down in the Rules following the procedures laid down in the Law on EIA.

77. The terms of the permit with the special part “Waste treatment (recovery or disposal, including storage and preparation for recovery or disposal)” must be adjusted in accordance with the procedure laid down in point 80.3 of the Rules if, after examining the information provided by the operator and reviewing the permit conditions, it is established that the planned changes to activities do not comply with the criteria referred to in Article 3(6) of the EIA Law and Article 192 of the Law of the Republic of Lithuania on Environmental Protection, but it is planned to amend:

77.1. non-hazardous waste:

77.1.1. the maximum quantity of total waste permitted to be stored simultaneously, including waste resulting from waste treatment;

77.1.2. maximum allowable total amount of waste to be disposed of;

77.1.3. waste management activities (type of activity code) in accordance with Annex 2 to the Waste Management Rules;

77.1.4. design capacity of the installation;

77.1.5. code(s) and name (names) of waste used or disposed of, including stored waste;

77.1.6. the maximum quantity of waste allowed on the site where it is produced before collection;

77.1.7. planned waste further treatment activities;

77.2. hazardous waste:

77.2.1. the maximum quantity of total waste permitted to be stored simultaneously in the technological stream, including the waste resulting from treatment;

77.2.2. maximum allowable amount of total waste to be disposed of in the technological stream;

77.2.3. management activities (type of activity code) in accordance with Annex 2 to the Waste Management Rules;

77.2.4. design capacity of the installation;

77.2.5. code(s) and name (names) of waste used or disposed of, including stored waste;

77.2.6. maximum quantity of waste on the site of production until collection;

77.2.7. planned waste further treatment activities;

78. Emission or release reduction measures shall be applied where, following a review of permit conditions carried out in accordance with the procedure laid down in paragraph 72 of the Regulations, the AAA determines that:

78.1. monitoring results or other data obtained (e.g. by reviewing permit conditions in the cases referred to in subparagraphs 72.1, 72.3, 72.4, 72.7 or 72.8 of the Rules) indicate that the pollution caused by the installation is permissible, but has significant adverse effects on the environment, and it is possible, in accordance with the procedure laid down by legal acts, to amend or add new emission or discharge limit values;

78.2. the safe operation of the installation requires the use of other technologies (e.g. after reviewing the permit conditions in the cases specified in paragraphs 72.3, 72.6 of the Rules).

79. When permit conditions are reviewed in accordance with point 72.6 of the Rules, the conditions of the permit are tightened or new (amending the permit or its conditions are modified), in the tables “Other conditions” in the specific parts, by specifying the technical/numerical parameters required by the information in the general part of the application required under point 25.1.6 of the Rules (if not included at the time of the permit), other conditions necessary to ensure compliance with the requirements of the legislation referred to in points 29.5, 32.2 to 1.2.9 of the Regulations, or other conditions permitting compliance with the environmental standards.

80. The AAA shall review the permit conditions within 20 working days of receipt of the information referred to in paragraphs 72.1 and 72.3 to 72.8 of the Rules or 1 years after the entry into force of the legislation referred to in paragraph 72.2 unless otherwise specified therein. In the case set out in paragraph 72.2 of the Rules, the AAA shall establish and approve a permit review plan for the revision of permit conditions. After reviewing the terms of the permit, the AAA shall take a decision on the obligation to amend the permit, write it on an AAA form and submit it to the operator and the AAD no later than 3 working days after the decision. The decision shall be taken in accordance with the following procedure:

80.1. if during the review of permit conditions at least one of the conditions for the permit change specified in Article 192 of the Law of the Republic of Lithuania on Environmental Protection is established, the AAA shall decide that it is necessary to amend the permit; The decision shall specify the parts of the permit, and on what basis, and set a time limit of at least 20 working days within which the operator must submit an application for renewal of a permit, where a permit is to be renewed in the cases specified in subparagraphs 4 and 5 of paragraph 9 of Article 192 of the Law of the Republic of Lithuania on Environmental Protection, a time limit of at least 20 working days, but not exceeding 30 working days. The decision must include the conditions with which an application for modification of an authorisation is to be submitted.

80.2. if the review of permit conditions does not establish any of the permit replacement clauses referred to in Article 192 of the Law of the Republic of Lithuania on Environmental Protection and the permit conditions in the special parts of the permit remain unchanged, the AAA shall decide that there is no need to amend the permit, specify in the decision the grounds on which it conducted the review of permit conditions and the reasons on which the decision was based;

80.3. if the review of permit conditions does not establish any of the permit replacement conditions specified in Article 192of the Law of the Republic of Lithuania on Environmental Protection, but it is established in accordance with the procedure laid down in the Rules that permit conditions must be adjusted, a decision shall be taken that the permit does not need to be amended. In that case, the decision shall specify the revised conditions of the authorisation: the name of the installation, the name of the special part of the permit, the table number and the name of the table, the conditions in force to be adapted to the conditions to be followed from the date of adoption of this Decision. The Decision states that this Decision is a permit condition and is to be taken into account in the operation of the installation and for the control of compliance with permit conditions. The decision shall be attached to the authorisation and shall be added to the list of additives to the authorisation.

81. Where the right to operate or control an entire installation or part of such an installation is acquired by another natural person, legal person or its branch (including a foreign legal person or another organisation, as well as a division thereof) or changes in the name, legal form, code, address (place of operation of the installation) of a natural person or of a legal person or other organisation (place of operation of the installation is not changed) or where the authorisation indicates a division of responsibility, the permit details shall be revised in accordance with the following provisions:

81.1. the successor/receivers in title of the authorisation holder or the holder of the authorisation together with the successor/transferee in title, if the responsibilities specified in the authorisation are shared, must notify the AAA in writing no later than 10 working days after the change in the data, and submit an application for revision of the permit details, documents supporting changes in the data and/or proof of the transfer of rights and division of responsibility, as appropriate, of the Rules 31.3, 32.3, or other documents for which the submission of an authorisation is required;

81.2. the AAA must revise the permit particulars no later than 10 working days from the date of receipt of the data and documents specified in paragraph 81.1 of the Rules. Where an incomplete application has been submitted or not all documents have been submitted or they do not meet the requirements laid down for them by legal acts, the time limit for updating the permit particulars shall be calculated from the date of receipt of all the data and/or documents required for revising the duly prepared permit to revise the particulars;

81.3. the decision to specify the permit details shall be made on the AAA form. The decision shall specify which person(s) shall, from the date of the decision, be considered to be the person(s) operating the installation to which the authorisation has been granted. Revisions to permit particulars shall replace the title sheet of the permit and, where appropriate, change the particulars of the permit in accordance with the AAA, the special parts of the authorisation being replaced. The registration number of the permit shall not be changed.

82. Where the operator is required to have financial security, the decision to adjust the permit particulars shall be taken when the legal successor (or successor) of the authorisation holder provides financial security to the AAA, i.e. when all duly documented documents are submitted in accordance with the requirements set out in point 81 of the Rules. When the operator has provided financial security, the AAA shall adjust the permit details within 10 working days from the date of the provision of the financial protection to the AAA.

83. When the Agency changes the maximum amount of waste permitted to be used, disposed of and/or the design capacity of the facility as specified in the permit, the AAA shall, within 5 working days, update the data in the Unitary Product, Packaging and Waste Accounting Information System.

CHAPTER VIII

REVOCATION OF AN AUTHORISATION

84. A permit shall be revoked in the presence of at least one of the conditions specified in paragraph11 of Article 192 of the Law of the Republic of Lithuania on Environmental Protection. In the case referred to in point 9 of this Part, where the NSC, in the performance of functions assigned to it by the Law of the Republic of Lithuania on Public Health in the field of smell management, detects violations of the permit conditions related to exceeding the limits of odour twice a year, the procedure for revocation of the AAA authorisation shall be initiated upon receipt of a notification from the NVSC containing a justification of the condition of withdrawal of the permit and inspection acts.

85. A decision to revoke an authorisation shall be taken in accordance with the following procedure:

85.1. prior to taking a decision to revoke the permit in the cases specified in subparagraphs 2, 3, 4, 6, 7, 8, 10 of paragraph 11 of Article 192 of the Law of the Republic of Lithuania on Environmental Protection, the AAA shall, within 3 working days from the establishment of the circumstances specified in these subparagraphs, warn the permit holder of a possible revocation of the permit and indicate the reasons why the permit may be revoked. In the cases referred to in Article 192 (11)( 2), (3), (4), (7) and (10) of the Law of the Republic of Lithuania on Environmental Protection, the AAA shall set a time limit of 20 working days within which a person must eliminate the specified circumstances in the case referred to in Article 192(11)(8) of the Law of the Republic of Lithuania on Environmental Protection, within which the person must eliminate the specified circumstances. If the holder of the authorisation removes the circumstances set out in the warning within this period and provides evidence from the AAA, the authorisation shall not be revoked. The AAA shall inform the authorisation holder within 5 working days of the non-revoking of the authorisation. In the case specified in Article 192 (11)( 6) of the Law of the Republic of Lithuania on Environmental Protection, the time limit within which a person must eliminate specified circumstances shall not be established;

85.2. In the cases referred to in subparagraphs 1, 5 and 9 of paragraph 11 of Article 192 of the Law of the Republic of Lithuania on Environmental Protection, the permit shall be revoked without giving a written warning;

85.3. the decision to revoke the validity of an authorisation shall be taken within the following time limits:

85.3.1. In the cases specified in subparagraphs 2, 3, 4, 7 and 10 of paragraph 11 of Article 192 of the Law of the Republic of Lithuania on Environmental Protection – within 3 working days from the expiry of the period of 20 working days specified in subparagraph 85.1 of the Rules;

85.3.2. In the case specified in subparagraph 8 of paragraph 11 of Article 192 of the Law of the Republic of Lithuania on Environmental Protection – within 3 working days from the expiry of the 6 month period referred to in subparagraph 85.1 of the Rules;

85.3.3. In the case specified in subparagraph 6 of paragraph 11 of Article 192 of the Law of the Republic of Lithuania on Environmental Protection – within 3 working days from the date of warning of the possible revocation of the permit provided in subparagraph 85.1 of the Rules;

85.3.4. In the cases referred to in point 85.2 of the Rules, within 3 working days from the date of establishment of the circumstances leading to withdrawal of the authorisation.

86. The authorisation shall be revoked by a reasoned decision of the AAA. The decision shall be made on an AAA form, stating the reasons for revocation of the authorisation and the procedure for appealing against the decision. The decision of the AAA shall, within 3 working days from the adoption of this Decision, be submitted to the operator (except when the permit has been revoked subject to the circumstances specified in Article 192 (11)(6) of the Law of the Republic of Lithuania on Environmental Protection), and in the event of revocation of the permit with the special part referred to in point 56.7 of the Rules, the NISSC.

CHAPTER IX

COMPLIANCE WITH PERMIT CONDITIONS AND CONTROL

87. The decision(s) referred to in paragraph 80 of the Rules shall be followed by the operator operating the authorised installation(s) and the AAD exercising control over compliance with permit conditions other than compliance with the conditions of the specific part of the permit. Compliance with the terms of the Special Part entitled ‘Quick Management’ of the authorisation shall be controlled by the NVSC.

88. Where, in the course of operation of an installation, the operator infringes the conditions of the permit, it must:

88.1. notify the APD within 3 working days;

88.2. take immediate measures to remedy the breach of permit conditions and to ensure that the permit conditions are complied with within the shortest possible period of time;

88.3. take the necessary additional measures as the ELD or EAA, within the scope of its competence, indicate as necessary in order to remedy the infringement and to restore compliance with permit conditions.

89. The NSC must, within 3 working days of the establishment of the conditions for withdrawal of the permit in paragraph 9 of Article 192 of the Law on Environmental Protection of the Republic of Lithuania relating to exceeding the limits on odour, notify the AAA of the obligation to withdraw the permit and submit the documentation justifying the establishment of these conditions.

CHAPTER X

PUBLICITY OF INFORMATION

90. The AAA shall publish on its website, no later than within 5 working days, the electronic versions of the issued permits or changes thereto (with the annexes referred to in paragraphs 55.4.6 and 55.4.7 of the Rules) and of the decisions of the AAA on the adjustment of permit conditions and withdrawal of authorisations. Authorisations with attachments referred to in paragraph 55.4 of the Rules shall be publicly available on the AAA website throughout the period of validity of the authorisations.

91. The AAA shall notify in writing the AAD, the Fire and Rescue Department and the executive institution of the municipality in the territory of which the activities are planned to be carried out of permits issued or modified within 3 working days from the date of adoption of the decision to issue or renew the permit by means of a link on the website of the AAA; if the authorisation is granted with the special part referred to in point 56.7 of the Rules, the NSC shall be informed in the same manner. The AAA shall inform the EAD of the revision of permit conditions or particulars no later than 3 working days from the date of the decision; if the authorisation is granted with the special part referred to in paragraph 56.7 of the Rules, it shall inform the NSC in the same manner. This information shall include the date of adoption of the decision, permit number, information, the procedure carried out (issued, modified permit, revised conditions, details, etc.), the installation’s address and intended start of activity.

CHAPTER XI

FINAL PROVISIONS

92. The operator shall have the right to appeal to the Administrative Disputes Commission or the court in accordance with the procedure laid down in the Law of the Republic of Lithuania on Administrative Proceedings.

93. In permits, errors shall be corrected in accordance with the legal grounds and conditions laid down in Article 35 of the Law of the Republic of Lithuania on Public Administration in accordance with the following procedure:

93.1. The AAA which issued or modified the authorisation and noticed or received from the operator or the control authority a written notification of calculation, printing, discrepancies in factual data or other technical errors which may have a material impact on compliance with the conditions laid down in the authorisation or on the authority’s control over compliance with permit conditions must correct errors;

93.2. the necessary condition for the recognition of an error is the fact that such correction will not prejudice the legitimate interests of third parties. Inaccurate and erroneous data may be corrected and corrected if the particulars entered in the authorisation granted or amended do not correspond to the documents on the basis of which they were entered;

93.3. Having completed the administrative rectification procedure, the AAA shall, within 3 working days, inform the operator and the ELD in writing of the errors, indicate them, send their corrected or new authorisation and publish information on past errors, their correction and the revised or new permit on its website.

Amendments to point.:

A note from the tar. Subparagraph 93 of the Rules for the Issuance, Replacement and Revocation of Pollution Permits shall enter into force on 1 January 2021.

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

94. Failure to comply with the requirements set forth in the Rules shall incur liability in accordance with the procedure laid down by legal acts of the Republic of Lithuania.

___

Rules on the granting, updating and revocation of pollution permits

Annex 1

CRITERIA FOR OPERATING AN INSTALLATION FOR WHICH A PERMIT IS REQUIRED

1. CRITERIA FOR OPERATING A PART OF THE SPECIAL PERMIT FOR WASTE WATER TREATMENT AND DISCHARGE

1.1. discharge (planned) to the natural environment (surface water bodies, filtering installations, fertilisation fields, etc.) 5 m³per day and more of household, industrial, etc. (except surface) waste water (calculated by dividing the amount of waste water discharged or to be discharged per year by the number of days of discharge);

1.2. discharges of surface water into the environment or planned to be discharged from the areas likely to be polluted, the surface waste water collection area of which exceeds 1 hectares (except car parks);

1.3. surface effluents shall be discharged into the environment, collected from surfaces of 10 ha and larger, intended for motor vehicles (streets, access points, parking areas) and/or where effluents from potentially polluted territories with a total surface waste water collection area exceeding 1 hectares are discharged into the common surface waste water treatment system;

1.4. discharge to the natural environment of waste water discharges in which, irrespective of the amount of waste water/debit, the concentration of priority substances is equal to or higher than the value indicated in Part A “Limit concentration in the natural environment” of Annex 2 to the Regulation on Waste Water Management approved by Order No D1-236 of the Minister of Environment of the Republic of Lithuania of 17May2006, and/or containing priority hazardous substances (regardless of the amount of priority hazardous substances released);

1.5. discharge from waste water from waste incineration or waste co-incineration plant from waste gas treatment;

1.6. discharges (planned) to surface water bodies of 5 m³per day and more (calculated by dividing the amount of water discharged or to be released per year from the number of release days) from fishing ponds.

2. CRITERIA FOR OPERATING INSTALLATIONS FOR WHICH PART OF THE SPECIFIC PERMIT IS REQUIRED FOR THE MANAGEMENT OF AMBIENT AIR POLLUTION

2.1. a combustion plant used with a rated thermal input equal to or greater than 1 MW but less than 50 MW and falling within the scope of the norms of emissions from medium combustion plants approved by Order No D1-778 of the Minister of Environment of the Republic of Lithuania of 18 September 2017 on the approval of norms of emissions from medium combustion plants;

2.2. the combustion plant used is within the scope of the Standards LAND 43-2013 of the emission standards for combustion plants approved by Order No D1-244 of the Minister of Environment of the Republic of Lithuania of 10 April 2013 on the Approval of Standards LAND 43-2013 for Emissions from Combustion Plants (hereinafter referred to as LAND 43-2013) and:

2.2.1. rated thermal input equal to or greater than 20 MW but less than 50 MW;

2.2.2. rated thermal input of less than 20 MW when this installation contains a solid fuel boiler of at least 0.5 MW rated (nominal) thermal capacity;

2.3. emissions to ambient air of 10 tonnes or more per year from stationary sources of pollution, with the exception of:

2.3.1. the sources of stationary pollution of installations used for the production of pollutants emitted to ambient air from farmed animals, the definition of which is given in the description of registration of places of keeping of farmed animals and marking and accounting of farmed animals kept therein, approved by Order No 3D-234 of the Minister of Agriculture of the Republic of Lithuania of 16 June 2003;

2.3.2.pollutants emitted into ambient air from combustion plants whose rated (nominal) thermal input is less than 20 MW and which fall within the scope of LAND 43-2013. The exemption shall not apply to the operation of combustion plants referred to in points 2.1 and 2.2 of this Annex;

2.3.3. pollutants emitted to ambient air from combustion plants whose rated (nominal) thermal input is less than 0.12 MW;

Emissions to ambient air from installations using solvents which:

2.3.4.1. according to the procedure for limiting emissions of volatile organic compounds due to the use of solvents in installations in certain types of activities, approved by Order No 620 of the Minister of Environment of the Republic of Lithuania of 5 December 2002 on the Approval of the Emission Restriction Procedure of Solvents due to the use of solvents in installations of certain activities (hereinafter referred to as the “Steel Procedure”), it must be registered, i.e. falling within the scope of the procedure for solvents due to the use of solvents in installations in Annex 1 to the procedure for the use of solvents, which is achieved by the use of solvents in Annex 1 of the Solvent Procedure;

2.3.4.2. excluded from the scope of the Solvent Procedure, i.e. the activity specified in Annex 1 to the Solvent Procedure does not reach the limit of solvent consumption set out in Annex 2 to the Solvent Procedure.

3. CRITERIA FOR OPERATING A SPECIFIC PERMIT FOR PART OF THE WASTE PERMIT (RECOVERY OR DISPOSAL, INCLUDING STORAGE AND PREPARATION FOR RECOVERY AND DISPOSAL)

3.1. waste treated (used or disposed of, including storage and preparation for recovery and disposal), except for the cases when in accordance with the Rules for Issuing, Amending and Revoking Pollution Prevention and Control Authorisations approved by Order No D1 —528 of the Minister of Environment of the Republic of Lithuania of 15 July 2013 on the Approval of the Rules for Issuing, Amending and Revoking Authorisations for Integrated Pollution Prevention and Control, Annex 1 requires an integrated pollution prevention and control permit;

3.2. waste stored on the site where it is produced exceeds the temporary storage period specified in the Law of the Republic of Lithuania on Waste Management;

3.3. storage of waste from extractive industries in waste facilities from the extractive industries.

4. CRITERIA FOR OPERATING A PART OF A SPECIFIC PERMIT FOR SURFACE WATER EXTRACTION OF ELIGIBLE INSTALLATIONS

Extraction or planned extraction of 100 m³per day (average annual collection) or more water from one body of surface water (with the exception of water used for amateur gardening needs and water used for hydropower production in hydropower plants whose output does not exceed 10 MW).

5. CRITERIA FOR OPERATING THE SPECIFIC PERMIT FOR PART OF THE CLIMATE CHANGE MANAGEMENT

A combustion plant with a rated thermal input exceeding 20 MW but less than 50 MW shall be used (except for installations for the incineration of hazardous or municipal waste).

6. CRITERIA FOR OPERATING A PART OF THE SPECIAL PERMIT FOR SHIP RECYCLING

Installations carrying out ship recycling activities as referred to in point (6) of Article 3(1) of Regulation (EU) No 1257/2013.

___

Rules on the granting, updating and revocation of pollution permits

Annex 1

Appendix 1

CRITERIA FOR OPERATING A SPECIFIC PART OF ODOUR MANAGEMENT FOR THE OPERATION OF THE INSTALLATIONS

Installations required to have at least one special part referred to in Annex 1 to the Regulations 1 to 6 and carrying out the following activities (weight of the packaging of manufactured products not included in the final weight of the finished production) are required to carry out the special part of the permit entitled ‘Flood Management’:

1. Treatment of biodegradable waste (except where only green waste is treated), extraction of biogas from waste, storage of sewage sludge.

2. Treatment of industrial waste water in waste water treatment plants.

3. Rearing of poultry or livestock.

4. Production of asphalt-concrete.

5. Production of wood panels (orientated strand board, shredded panels or fibreboard) with a production capacity of at least 300 m³per day but not exceeding 600 m³ per day.

6.Manufacture of plastics, including foam, manufacture of plastic products from pellets with a production capacity of 5 tonnes or more per day.

7.Manufacture of vegetable or animal fats, oils and products thereof, manufacture of bread and confectionery with a production capacity of 5 tonnes or more per day.

8. Sugar production.

9. Production of beer with a production capacity of 5 litres or more per day.

10. Production of feed of animal origin with a production capacity of 35 tonnes or more per day.

11. Processing of meat, slaughter of animals or birds with a production capacity of not less than 25 tonnes per day but not exceeding 50 tonnes per day.

12. Oil, petroleum products storage and handling, water management.

13. Tanning of hides and skins with a treatment capacity of 6 tonnes or more but not more than 12 tonnes of finished products per day.

14. Handling of animal by-products

Amendments to the Annex:

A note from the tar. Appendix 1 of Annex 1 to the Rules for Issuing, Replacement and Withdrawal of Pollution Permits shall enter into force on 1 January 2021.

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

Rules on the granting, updating and revocation of pollution permits

Annex 1

Appendix 2

|  | ACTIVITIES EXEMPTED FROM THE REQUIREMENT TO HOLD THE PART OF THEPERMIT”TREATMENT (RECOVERY OR DISPOSAL, INCLUDING STORAGE AND PREPARATION FOR RECOVERY OR DISPOSAL)” (FOR ESTABLISHMENTS WHICH USE OR DISPOSE OF ON SITE, INCLUDING PREPARATION FOR RECOVERY OR DISPOSAL, ONLY NON-HAZARDOUS WASTE GENERATED BY THEIR OWN ECONOMIC ACTIVITIES) No; Economic activities and codes; Name of waste; Waste code; Quantity of waste, t/month; Waste recovery and disposal activities. Codes; Name of activity; Brief description of activities. 1.; Crop and livestock production, hunting and related service activities (A01); Vegetable tissue waste; 02 01 03; ≤ 500; R10 R3; Land treatment which is beneficial to agriculture or improves the state of the environment. Recycling/reclamation of organic substances not used as solvents (including composting and other biological substitution processes).; Operators producing cereals or other plants composting or treating anaerobically produced waste of vegetable origin (such as cereals) and fertilising their fields with the resulting compost or anaerobic rasp. Farmers composte plant-based waste and fertilise their fields with compost.. 2.; Forestry and logging (A02); Forestry wastes; 02 01 07; ≤ 900; R1 R3; Use for the production of fuel or other means of energy recovery. Recycling/reclamation of organic substances not used as solvents (including composting and other biological substitution processes); Logging companies use wood waste as fuel in their boiler houses. Composting of various branches of trees and other forestry waste.. 3.; Manufacture of wood and articles of wood and cork, except furniture, manufacture of articles of straw and plaiting materials (C16); Wood bark and wood waste; 03 03 01; ≤ 1000; R1; Use for the production of fuel or other means of energy recovery.; Wood processing companies use wood waste as fuel in their boiler houses.. Sawdust, shavings, chips, wood, particle board and plywood, not specified 03 01 04; 03 01 05; ≤ 1300; R1. 4.; Manufacture of rubber and plastic products (C22); Plastic waste; 07 02 13; ≤ 800; R3; Recycling/reclamation of organic materials not used as solvents; Plastic waste produced during production is reused to produce products.. Plastic shavings and trimmings; 12 01 05; ≤ 800; R3; Recycling/reclamation of organic materials not used as solvents. 5.; Manufacture of other non-metallic mineral products (C23); Alloy of dolomite and basalt; 01 04 99 10 01 99; ≤ 1800 ≤ 1800; R5; Recycling/reclamation of other inorganic materials; Waste from stone wool production is reused for production.. Gravel and crushed waste other than those mentioned in 01 04 07; 01 04 08; ≤ 100; R10; Land treatment beneficial to agriculture or improving the state of the environment; Quarries are used to re-cultivate quarries.. Particulates and dust (excluding 10 13 12 and 10 13 13); 10 13 06; ≤ 10; R5; Recycling/reclamation of other inorganic materials; Waste from cement production is used for the production of cement.. 6.; Manufacture of furniture (C31); Sawdust, shavings, chips, wood, particle board and plywood, not specified 03 01 04; 03 01 05; ≤ 1300; R1; Use for the production of fuel or other means of energy recovery.; The waste generated during the production process is used for fuel in its own boiler houses.. 7.; Construction of buildings (F41), Engineering works (F42), Specialised construction activities (F43); Construction and demolition wastes: concrete bricks mixtures of concrete, bricks, tiles and ceramics (except 17 01 06) bituminous mixtures (except 17 03 01) for earth and stones (except 17 05 03), road splitting (except 17 05 07).; 17 01 01 17 01 02 17 01 03 17 01 07 17 03 02 17 05 04 17 05 08; ≤ 400; R5 R12; Recycling/reclamation of other inorganic materials Change in the status or composition of waste prior to any of the R1-R11 activities; Shredded construction and demolition waste used in the Construction Waste Management Rules approved by Order No. D1-637 of the Minister of Environment of the Republic of Lithuania of 29 December 2006 on the Approval of Construction Waste Management Rules, or used as construction products, when their conformity is confirmed by the declaration of performance by the producer of these products Sorting by type and/or fraction of waste for further use. 8.; Wholesale trade, except of motor vehicles and motorcycles (G46); Paper and cardboard packaging; 15 01 01; ≤ 30; R12; Change in the status or composition of waste prior to any of the R1-R11 activities; Packaging waste is constrained and transferred for further handling.. 9.; Retail sale, except of motor vehicles and motorcycles (G47); Plastic packaging without PET (polyethylene film); 15 01 02; ≤ 2; R12; Change in the status or composition of waste prior to any of the R1-R11 activities; Packaging waste is constrained and transferred for further handling.. 10.; Accommodation and food service activities (I55) (I56) **; Bio-waste (food and kitchen waste) Bio-waste (sod and park green waste); 20 01 08 20 02 01; ≤ 0,400; R10 R3; Land treatment which is beneficial to agriculture or improves the state of the environment. Recycling/reclamation of organic substances not used as solvents (including composting and other biological substitution processes).; Economic operators (e.g. hotels, motels, restaurants, catering establishments (e.g. educational establishments) compostable biological waste from economic activities (food and kitchen, green waste) and use compost.. *Note: the names and codes of economic activities are indicated according to the Classification of Economic Activities approved by Order No DD-226 of the Director General of the Department of Statistics of 31 October 2007 on the Approval of the Classification of Economic Activities ** Applicable to other economic entities using only biological waste generated in the course of their economic activities up to 5 t/year. |
| --- | --- |

Rules on the granting, updating and revocation of pollution permits

Annex 2

(Model application)

APPLICATION

TO OBTAIN OR RENEW A PERMIT

(Legal personal number)

(Name, home address, telephone, fax number, e-mail address of the operator applying)

(name and address of the object of the economic activity)

(specify which criteria according to Annex 1 of the Regulations the equipment meets)

(contact details, telephone, fax number, e-mail address)

I. general PARTIES

(information according to item 25 of the Rules)

Use of raw materials, fuels and chemicals in production

Table 1:The plant uses raw materials, fuels and additional substances.

| Serial No | Name of raw material, fuel type or material | Estimated amount used, measurement units (t, m³or other per year) | Quantity stored at the same time (t, m³or other per year), method of storage (open site or tanks, closed containers or covered site, etc.) |
| --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |

Table2:Hazardous substances and chemical mixtures used in the installation

| General information on the substance or mixture |  |  | Information on the hazardous substance (pure or contained in the mixture) |  |  |  |  | Storage, use, recovery |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| The salesman’s substitute— dyspnoea | Material— GA or mixture | Date of establishment of the Safety Data Sheet(SDL) | Um... the material of the material— the replacement of the GIS— dyspnoea | Concent— information in the mixture | EC and CAS No -, | Hazard Class and Category according to Classification and Labelling Regulation 1272/2008 | The risk— of rubber phrase | Quantity (t) stored at the same time and method of storage | Annual consumption(t) | Where the production is used— y | Determined (calculated) emissions or discharges of a substance | Utili-zavi-method |

II.SPECIAL PARTS

III.ANNEXES TO THE APPLICATION, OTHER INFORMATION AND PARTICULARS REQUIRED BY THE RULES

Rules on the granting, updating and revocation of pollution permits

Annex 2

Appendix 1

SPECIFIC PART OF THE APPLICATION

Waste water management and discharge

Table1:Information on the surface water body (receiver) into which waste water is planned to be discharged

| Serial No | Name, category and code of the water body | 80 % probability average flow rate, m³/s(rivers) | Water body area, ha (for stable water bodies) | Status of the body of water |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Indicator | Current (background) status |  | Permissible load on a water body |  |  |
|  |  |  |  |  | units of Measurement. | value | Hydraulic, m³/d. | pollutants |  |
|  |  |  |  |  |  |  |  | units of Measurement. | value |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |

Table2:Information about the waste water discharge/receiver to which waste water is planned to be discharged, when the waste water is planned to be infiltrated into the ground in filtration facilities installed for that purpose, accumulate in storage tanks during periodic transportation, etc.

| Serial No | Description of effluent discharge/receiver | Legal Waste Water issue of grounds | #### Allowable load of the receiver |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | #### hydraulic | #### pollutants |  |  |
|  |  |  | M3/d | parameter | units of Measurement. | value |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |

Table3:Data on waste water sources and/or dischargers

| No -, | Coordinates | Number of receiver | Description of the effluent to be discharged | Release device type/technical data | Description of release device location | Maximum amount of effluent to be discharged |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | M3/d | M3/m |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |

Table
4:Pollution of the effluent planned to be discharged to the natural environment

| No -, | Name of the pollutant | Waste water contamination before treatment |  |  | Maximum desired contamination of waste water when discharged into the environment |  |  |  |  |  |  |  | Cleaning effect: % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | mom. mg/l | in the middle, mg/l | t/year | MRL mom. mg/l | Desired LK mom., mg/l | MRL average, mg/l | Page-given LK ind., mg/l | Days of DLT, t/d. | Desired LT days, t/d. | Years of DLT, t/m. | Desired LT year, t/m. |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |

Table 5:Waste water and pollution reduction measures used at the facility/installation

| Serial No | Waste water source/discharger | Description of the measure and its purpose | Date of installation | Design characteristics of the measure |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | indicator | units of Measurement. | value |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |

Table6:List of industrial enterprises and other subscribers from which waste water is planned to be taken and characteristics of the waste water to be adopted

| Serial No -, | Name of subscriber | Maximum amount of waste water expected to be taken from the subscriber | Maximum pollution expected to be generated by subscriber’s waste water |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Thousand m³/m | Pollutants | LKmom., mg/l | LKaverage., mg/l | LTday, t/d | LTannual, t/year |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1. | Subscribers intended to receive waste water (except surface waters) contaminated with priority and/or priority hazardous substances shall: |  |  |  |  |  |  |
| 1.1. |  |  |  |  |  |  |  |
| 1.2. |  |  |  |  |  |  |  |
| 2. | Subscribers intended to receive more than 50 m³/day of industrial waste water but who do not meet the criteria set out in point 1: |  |  |  |  |  |  |
| 2.1. |  |  |  |  |  |  |  |
| 2.2. |  |  |  |  |  |  |  |
| 3. | Aggregated data of subscribers from which production waste water is to be taken (which do not meet the criteria set out in points 1 and 2): |  |  |  |  |  |  |
| 4. | Aggregated data from other subscribers (which do not meet the criteria set out in points 1, 2 and 3): |  |  |  |  |  |  |
| 5. | Total (data for all expected receipts from subscribers): |  |  |  |  |  |  |

Table7:List ofindustrial enterprises and other subscribers intended to receive surface effluent and properties of waste water planned to be adopted

| Serial No -, | Subscriber | Maximum amount of waste water expected to be taken from the subscriber | Maximum pollution expected to be generated by subscriber’s waste water |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Thousandm³/m | Pollutants | Lk mom. mg/l | Lk midway, mg/l | LT days, t/d. | EN annual, t/m. |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 6. | The following surface waste water is to be collected from subscribers intended to receive from the areas potentially polluting as defined in the Surface Waste Water Treatment Regulation: |  |  |  |  |  |  |
| 6.1. |  |  |  |  |  |  |  |
| 6.2. |  |  |  |  |  |  |  |
| 7. | Aggregate surface water discharges by other subscribers (which do not meet the criteria set out in row 6): |  |  |  |  |  |  |
| 8. | Total (for all subscribers referred to in rows 6 and 7 expected to receive waste water): |  |  |  |  |  |  |

Rules on the granting, updating and revocation of pollution permits

Annex 2

Appendix 2

SPECIFIC PART OF THE APPLICATION

AMBIENT AIR POLLUTION MANAGEMENT

Table1: Pollutants to ambient air expected to be emitted to ambient air

| Name of the pollutant | Pollutant code | Intended to be discarded (requested) in t/m. |
| --- | --- | --- |
| 1 | 2 | 3 |
| Oxides of nitrogen |  |  |
| Particulate matter |  |  |
| Sulphur dioxide |  |  |
| Ammonia |  |  |
| Volatile organic compounds (in alphabetical order): |  |  |
| Other pollutants (in alphabetical order): |  |  |
|  | Total: |  |

Table2:Data onstationary sources of ambient air pollution

Installation name___.

Economic activity code according to the Classification of Economic Activities (EVRK Rev. 2)___

| Sources of Pollution |  |  |  | Exhaust gas indices at the sampling/measuring point |  |  | The duration of emissions (of stationary sources), hours/m/m |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No -, | coordinates | height, m | output dimensions, m | the speed of the flow, m/s | temperature, °C | volume flow rate, m3/s |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |

Table 3: Pollution into ambient air

Installation name___

| Name or No of the workshop or other | Sources of Pollution |  | Pollutants |  | Expected (requested for authorisation) pollution |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | No -, |  | title | code | single use only size |  | annual, t/m. |
|  |  |  |  |  | units | Max. |  |
| 1 | 2 |  | 3 | 4 | 5 | 6 | 7 |
|  |  |  |  |  | Total installation: |  |  |

Table4:Installations for ambient air pollution and pollution prevention measures

Name of installation

| No of the emission source to which the gas flow through the purification plant enters. | Treatment plants | Pollutants cleaned/decontaminated in the cleaning plant |  |
| --- | --- | --- | --- |
|  | name and description of destination | title | code |
| 1 | 2 | 3 | 4 |
| Pollution prevention measures shall be as follows: |  |  |  |

Table 5:Pollution into ambient air under unusual (extraordinary) operating conditions

Installation name___

| Pollution source from which emissions occur under these conditions | Conditions that may result in abnormal (non-conventional) emissions | Details of unusual (non-conformity) emissions data |  |  |  |  | Remarks describing in more detail the repetition, duration, etc. of abnormal emissions |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | the duration of the discharges, min. (emphasis added) | pollutants |  | Concentration of pollutants in the exhaust, mg/Nm3 |  |
|  |  |  |  | title | code |  |  |
| 1 | 2 |  | 3 | 4 | 5 | 6 | 7 |

Rules on the granting, updating and revocation of pollution permits

Annex 2

Appendix 3

SPECIFIC PART OF THE APPLICATION

CLIMATE CHANGE MANAGEMENT

Greenhouse gas emissions (GHG) from combustion plants with a total rated thermal input exceeding 20 MW but less than 50 MW (excluding installations for the incineration of hazardous or municipal waste)

| Serial No | GHG (carbon dioxide (CO2)) emission sources |
| --- | --- |
| 1 | 2 |

Rules on the granting, updating and revocation of pollution permits

Annex 2

Appendix 4, Part A

SPECIFIC PART OF THE APPLICATION

TREATMENT OF WASTE (RECOVERY OR DISPOSAL, INCLUDING STORAGE AND PREPARATION FOR RECOVERY OR DISPOSAL)

WASTES NOT COVERED

Table 1:Maximum amount of non-hazardous waste to be stored

Name of installation

| Waste |  |  | Storage of waste |  | Planned further treatment of waste |
| --- | --- | --- | --- | --- | --- |
| Code | Title | Revised name | Waste management activity code (R13 and/or D15) | Maximum total amount of waste, including waste generated by treatment, to be stored simultaneously, t |  |
| 1 | 2 | 3 | 4 | 5 | 6 |

Table 2: Maximum expected quantity of non-hazardous waste on site prior to collection (S8)

Name of installation

| Waste |  |  | Storage of waste | Planned further treatment of waste |
| --- | --- | --- | --- | --- |
| Code | Title | Revised name | Maximum total amount of waste to be stored at the same time, t |  |
| 1 | 2 | 3 | 4 | 5 |

Table 3: Non-hazardous waste intended to be used, except those intended to be stored and prepared for recovery

Name of installation

| Wastes to be used, other than those to be stored and prepared for recovery |  |  | Recovery of waste |  | Planned further treatment of waste |
| --- | --- | --- | --- | --- | --- |
| Code | Title | Revised name | Waste recovery activity code (R1-R11) | Design capacity of the installation, t/m |  |
| 1 | 2 | 3 | 4 | 5 | 6 |

Table 4: Non-hazardous waste to be disposed of, with the exception of non-hazardous waste intended to be disposed of and prepared for disposal

Name of installation

| Wastes to be disposed of other than those to be stored and prepared for disposal |  |  | Disposal of waste |  |  |
| --- | --- | --- | --- | --- | --- |
| Code | Title | Revised name | Waste disposal activity code (D1–D7, D10) | Design capacity of the installation | Maximum expected total amount of waste to be disposed of, t/m |
| 1 | 2 | 3 | 4 | 5 | 6 |

Table 5: Non-hazardous waste to be prepared for recovery and/or disposal

Name of installation

| Waste to be prepared for recovery and/or disposal |  |  | Preparation of waste for recovery and/or disposal |  |
| --- | --- | --- | --- | --- |
| Code | Title | Revised name | Waste management activity code (D8, D9, D13, D14, R12, S5) | Design capacity of the installation, t/m |
| 1 | 2 | 3 | 4 | 5 |

6. Other information in accordance with paragraph 32.2 of the Rules.

___

Rules on the granting, updating and revocation of pollution permits

Annex 2

Appendix 4, Part B

SPECIFIC PART OF THE APPLICATION

TREATMENT OF WASTE (RECOVERY OR DISPOSAL, INCLUDING STORAGE AND PREPARATION FOR RECOVERY OR DISPOSAL)

HAZARDOUS WASTE

Table 1:Maximum amount of hazardous waste to be stored

Name of installation

| Labelling of the technological stream of hazardous waste | Name of the technological stream of hazardous waste | Waste code | Name of the waste | Revised name of waste | Storage of waste |  | Planned further treatment of waste |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Waste management activity code (R13 and/or D15) | Maximum total amount of waste, including waste generated by treatment, to be stored simultaneously, t |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |

Table 2: Maximum expected amount of hazardous waste on site prior to collection (S8)

Name of installation

| Labelling of the technological stream of hazardous waste | Name of the technological stream of hazardous waste | Waste code | Name of the waste | Revised name of waste | Storage of waste | Planned further treatment of waste |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Maximum expected to be stored at the same time total quantity of waste, t |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |

Table 3: Hazardous waste to be used, with the exception of hazardous waste intended to be stored and prepared for recovery

Name of installation

| Dangerous marking of the waste technological stream | Name of the technological stream of hazardous waste | Waste code | Name of the waste | Revised name of waste | Recovery of waste |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Waste recovery activity code (R1-R11) | Design capacity of the installation, t/m | Planned further treatment of waste |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |

Table 4: Hazardous waste to be disposed of, with the exception of hazardous waste intended to be stored and prepared for disposal

Name of installation

| Labelling of the technological stream of hazardous waste | Name of the technological stream of hazardous waste | Waste code | Name of the waste | Revised name of waste | Disposal of waste |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Waste disposal activity code (D1–D7, D10, D12) | Design capacity of the installation | Maximum expected total amount of waste to be disposed of, t/m |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |

Table 5: Hazardous waste to be prepared for recovery and/or disposal

Name of installation

| Dangerous marking of the waste technological stream | Name of the technological stream of hazardous waste | Waste code | Name of the waste | Revised name of waste | Preparation of waste for recovery and/or disposal |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Waste management activity code (D8, D9, D13, D14, R12, S5) | Design capacity of the installation, t/m |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |

Other information in accordance with paragraph 32.2 of the Rules.___

Rules on the granting, updating and revocation of pollution permits

Annex 2

Appendix 5

SPECIFIC PART OF THE APPLICATION

EXTRACTION OF WATER FROM SURFACE WATER BODIES

Table 1:Information on the surface water body from which the water is to be extracted, the point of abstraction and the amount of water to be extracted. The table shall be completed if it is intended to extract water from surface water bodies, taking into account the criteria specified in Annex 1 to the Regulations.

| Category of water body (river, lake, pond, etc.) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name of body of water |  |  |  |  |  |  |
| Water body identification code |  |  |  |  |  |  |
| 80 % probability average river flow in m³/s for the dryest month |  |  |  |  |  |  |
| Volume of lake, pond, m³ |  |  |  |  |  |  |
| Water extraction site in respect of protected areas |  |  |  |  |  |  |
| Water extraction point No |  |  |  |  |  |  |
| Coordinates of the water extraction point |  |  |  |  |  |  |
| Maximum amount of water to be extracted | m3/m | m3/p. | m3/m | m3/p. | m3/m | m3/p. |

Rules on the granting, updating and revocation of pollution permits

Annex 2

Appendix 6

SPECIFIC PART OF THE APPLICATION

SHIP RECYCLING

Table 1:Information on the ship recycling facility

Installation name___

| Ship recycling method(s) | Expected type and size of the ship (length, width, depth, GT) for the most recyclable installation | Maximum quantity of vessels intended for recycling in the installation to be stored simultaneously (i.e., units, etc.) | Maximum planned amount of vessels to be recycled in the installation, units of measurement (i.e., units, etc.) | Maximum annual ship recycling capacity of the installation (LDT/year) |
| --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 |

Table 2:Information on waste resulting from ship recycling (applies to installations where the waste generated at the place where it is produced is kept for less than the temporary storage period laid down in the Law of the Republic of Lithuania on Waste Management and which does not carry out activities related to the treatment of waste)

Installation name___

| Waste |  |  | Storage of waste | Planned further treatment of waste |
| --- | --- | --- | --- | --- |
| Code | Title | Revised name | Expected maximum quantity of waste stored at the same time, units of measurement (t, m³ orother) |  |
| 1 | 2 | 3 | 4 | 5 |

Table 3:Information on hazardous substances contained in the structure and/or equipment of the ship to be recycled, including those referred to in Annexes I and/or II to Regulation (EU) No 1257/2013 (applicable to installations processing vessels of 500 GT and above. Installations processing ships of less than 500 GT are intended to be recycled in the case of ships with a structure and/or equipment containing dangerous substances)

Installation name___

| Name of the hazardous substance | Quantity stored on site (t, m³or other) | Intended storage/storage method | Intended method of treatment |
| --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |

Rules on the granting, updating and revocation of pollution permits

Annex 2

Appendix 7

SPECIFIC PART OF THE APPLICATION

ODOR MANAGEMENT

Table1:Data onstationary sources of odour

| Source of smell |  |  |  |  | Exhaust gas indices at sampling/measuring site |  |  | Odour emission rate*, OUE/s, OUE/m/s, OUE/m²/s,OUE/m³/s | Daily/week/year of emissions (action of stationary sources) with specific hours |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Odour source No | title | coordinates (perimeter coordinates of the field source) (LCS) | height from the ground, m | output dimensions, m | the speed of the flow, m/s | temperature t, °C | volume flow rate, Nm3/s |  |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |

* Coffee emission indicator is defined in the Rules of the Quality Control in the Living Environment, approved by Order No. V-885 of the Minister of Health of the Republic of Lithuania on the approval of the Lithuanian Hygiene Norm HN 121: 2010 “Limit value of concentration in living environment” and the Rules of Forest Control in Living Environment.

Table 2: Odor management/reduction measures, their efficiency indicators

| Odour source No | Measures for the management/reduction of odour |  |  | Expected (requested) odour emission indicator OUE/s, OUE/m/s, OUE/m²/s, OUE/m³/s |
| --- | --- | --- | --- | --- |
|  | title | installation location, coordinates, LKS | efficiency, % |  |
| 1 | 2 | 3 | 4 | 5 |

Table 3:Effectiveness of odor management (reducing) measures at closest receptors

| Measured odour concentration (OUE/m³)to the nearest sensitive receptor* | Address and coordinates of the nearest sensitive receptor (LKS) |
| --- | --- |
| 1 | 2 |
|  | “ |

*sensitive receptors – a structure or territory in which people live, rest, or are temporarily sensitive groups of society (children, patients, etc.), e.g. residential, kindergarten, school, hospital, sanatorium, recreation home, residential or recreational areas, etc.

Amendments to the Annex:

A note from the tar. Appendix 7 of Annex 2 to the Rules for Issuing, Replacement and Withdrawal of Pollution Permits shall enter into force on 1 January 2021.

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

Rules on the granting, updating and revocation of pollution permits

Annex 2

Appendix 8

(example of declaration)

declaration

I apply for a permit/replacement of pollution permits.

I certify that the information provided in this application is correct, complete and accurate.

I am without prejudice to the submission by the authorising authority to third parties of a copy of the application or part thereof, except for the information referred to in that application as a commercial/industrial secret.

Signature: Date of ___Date	: ___

(operator or person authorised by him)

___

(name and capacity of signator)

___

Rules on the granting, updating and revocation of pollution permits

Annex 2

Appendix 9

(example of declaration)

declaration

I undertake to declare, report and surrender, by 30 April each calendar year, the amount of greenhouse gas emissions in the previous calendar year equivalent to the amount of carbon dioxide emissions and/or carbon dioxide equivalents in tonnes and/or carbon dioxide equivalent in the previous calendar year.

Signature: Date of ___Date	: ___

(operator or person authorised by him)

___

(name of signatory, capacity (to be completed in capital letters)

___

Rules on the granting, updating and revocation of pollution permits

Annex 2

Appendix 10

(example of declaration)

DECLARATION

ABOUT THE NUMBER OF OPERATING HOURS

I hereby certify that an average combustion plant with:

Type: ___

Location of operation (address): ___

Date of commencement of operation: ___

Rated thermal input: ___

it will not work more than ___ operating hours (as they are defined in the norms of emissions from medium combustion plants approved by Order No. D1-778 of the Minister of Environment of the Republic of Lithuania of 18 September 2017 on the approval of norms of emissions from medium combustion plants) per year (using the rolling average of ___).

I undertake to keep data on the number of operating hours per year for at least six consecutive calendar years.

Signature: ___ Date: ___

(operator or person authorised by him)

(name and capacity of signator (to be completed in capital letters)

___

Rules on the granting, updating and revocation of pollution permits

Annex 3

(example of pollution permit)

___

(name of issuing or amending authority (in capitals))

PERMIT

No___

(Legal personal number)

(name and address of the object of the economic activity)

(operator, home address, telephone, fax number, e-mail address)

The authorisation shall consist of:

1. Special part(s): ......................................................................................................

2. Registration number of IPPC permit in force prior to the renewal of the permit (if issued), name of the regional environmental department which issued it, date of issue, renewal and/or correction (if any): ......................................................

3. Additives to the authorisation

Issued.....................................................

Amended by...................................

The Director or a person authorised by him ___ ___

(Name)	 (signature)

A. V.

GENERAL PART

(information according to item 59 of the Rules)

Rules on the granting, updating and revocation of pollution permits

Annex 3

Appendix 1

SPECIAL PART OF THE PERMIT

Waste water management and discharge

Table1:Waste water discharge/receiver information

| Serial No | Effluent point/receiver name, category, code and coordinates | #### Allowable load of the receiver |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | #### hydraulic | #### pollutants (in units of weight) |  |  |
|  |  | m3/d | parameter | units of Measurement. | value |
| 1 | 2 | 3 | 4 | 5 | 6 |

Table2:Pollution of effluent permitted to be discharged into the natural environment. Emission limit values set shall not apply in the event of abnormal (non-conformity) operating conditions – starting, combining and stopping an installation

| No -, | Name of the pollutant | Waste water contamination before treatment |  |  | Maximum allowable contamination of waste water by discharge into the environment |  |  |  | Minimum cleaning efficiency, % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | mom. mg/l | In the middle, mg/l | t/year | Lk mom. mg/l | Stay in the middle. mg/l | LT days, t/d. | Years, t/m. |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |

Table3:Pollution of the discharged waste water. Emission limit values set shall apply in the event of abnormal (non-conformity) operating conditions – starting, combining and stopping an installation and a specific period of implementation of environmental protection measures

| No -, | Name of the pollutant | Waste water contamination before treatment |  |  | Maximum desired contamination of waste water when discharged into the environment |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | mom. mg/l | in the middle, mg/l | t/year | LLK mom., mg/l | LLK average, mg/l | Days of LLT, t/d. | Year of LLT, t/m. |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |

Table4:Other conditions for waste water treatment and discharge in accordance with the requirements of legislation on environmental protection

| Serial No | Conditions |
| --- | --- |
| 1 | 2 |

Rules on the granting, updating and revocation of pollution permits

Annex 3

Appendix 2

SPECIAL PART OF THE PERMIT

AMBIENT AIR POLLUTION MANAGEMENT

Table1:Allowable Emissions to Ambient Air and their Amount

| Name of the pollutant | Pollutant code | Discarded, t/m. |
| --- | --- | --- |
| 1 | 2 | 3 |
| Oxides of nitrogen |  |  |
| Particulate matter |  |  |
| Sulphur dioxide |  |  |
| Ammonia |  |  |
| Volatile organic compounds (in alphabetical order): |  |  |
| Other pollutants (in alphabetical order): |  |  |
|  | Total: |  |

Table2:Pollution into ambient air shall be permitted.(Emission limit values set do not apply during abnormal/non-conforming operating conditions)

Installation name___

| Name or No of the workshop or other | Sources of Pollution |  | Pollutants |  | Permitted pollution |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | No -, |  | title | code | single use only size |  | annual, t/m. |
|  |  |  |  |  | units | Max. |  |
| 1 | 2 |  | 3 | 4 | 5 | 6 | 7 |
|  |  |  |  |  | Total installation: |  |  |

Table3:Other conditions for the management of ambient air pollution in accordance with the requirements of legal acts on environmental protection

| Serial No | Conditions |
| --- | --- |
| 1 | 2 |

Rules on the granting, updating and revocation of pollution permits

Annex 3

Appendix 3

SPECIAL PART OF THE PERMIT

CLIMATE CHANGE MANAGEMENT

Greenhouse gas emissions from combustion plants with a total rated thermal input exceeding 20 MW but less than 50 MW (excluding installations for the incineration of hazardous or municipal waste)

| Serial No | GHG (carbon dioxide (CO2)) emission sources |
| --- | --- |
| 1 | 2 |

Rules on the granting, updating and revocation of pollution permits

Annex 3

Appendix 4, Part A

SPECIAL PART OF THE PERMIT

TREATMENT OF WASTE (RECOVERY OR DISPOSAL, INCLUDING STORAGE AND PREPARATION FOR RECOVERY OR DISPOSAL)

WASTES NOT COVERED

Table 1:Maximum quantity of non-hazardous waste to be stored

Name of installation

| Waste |  |  | Storage of waste |  | Further treatment of waste |
| --- | --- | --- | --- | --- | --- |
| Code | Title | Revised name | Waste management activity code (R13 and/or D15) | Maximum quantity of waste, including waste resulting from treatment, to be stored simultaneously, t |  |
| 1 | 2 | 3 | 4 | 5 | 6 |

Table 2: Maximum quantity of non-hazardous waste allowed on the site prior to collection (S8)

Name of installation

| Waste |  |  | Storage of waste | Further treatment of waste |
| --- | --- | --- | --- | --- |
| Code | Title | Revised name | Maximum total quantity of waste allowed at the same time, t |  |
| 1 | 2 | 3 | 4 | 5 |

Table 3: Non-hazardous wastes authorised for use, other than those intended to be stored and prepared for recovery

Name of installation

| Waste |  |  | Recovery of waste |  | Further treatment of waste |
| --- | --- | --- | --- | --- | --- |
| Code | Title | Revised name | Waste recovery activity code (R1-R11) | Design capacity of the installation, t/m |  |
| 1 | 2 | 3 | 4 | 5 | 6 |

Table 4: Non-hazardous wastes authorised for disposal other than those to be stored and prepared for disposal

Name of installation

| Waste |  |  | Disposal of waste |  |  |
| --- | --- | --- | --- | --- | --- |
| Code | Title | Revised name | Waste disposal activity code (D1–D7, D10) | Design capacity of the installation | Maximum allowable total amount of waste to be disposed of, t/m |
| 1 | 2 | 3 | 4 | 5 | 6 |

Table 5: Permitted non-hazardous waste prepared for recovery and/or disposal

Name of installation

| Waste |  |  | Preparation of waste for recovery and/or disposal |  |
| --- | --- | --- | --- | --- |
| Code | Title | Revised name | Waste management activity code (D8, D9, D13, D14, R12, S5) | Design capacity of the installation, t/m |
| 1 | 2 | 3 | 4 | 5 |

Table6:Obligation to have effective financial security where this requirement is applied in compliance with theLaw of the Republic of Lithuania on Waste Management and other conditions for the treatment (recovery or disposal, including storage and preparation for recovery or disposal) of non-hazardous waste under the requirements of legal acts regulating environmental protection

| Serial No | Conditions |
| --- | --- |
| 1 | 2 |

___

Rules on the granting, updating and revocation of pollution permits

Annex 3

Appendix 4, Part B

SPECIAL PART OF THE PERMIT

TREATMENT OF WASTE (RECOVERY OR DISPOSAL, INCLUDING STORAGE AND PREPARATION FOR RECOVERY OR DISPOSAL)

HAZARDOUS WASTE

Table 1:Maximum quantity of hazardous waste to be stored

Name of installation

| Labelling of the technological stream of hazardous waste | Name of the technological stream of hazardous waste | Waste code | Name of the waste | Revised name of waste | Storage of waste |  | Further treatment of waste |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Waste management activity code (R13 and/or D15) | Maximum quantity of waste, including waste resulting from treatment, to be stored simultaneously, t |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |

Table 2: Maximum quantity of hazardous waste on site prior to collection

Name of installation

| Labelling of the technological stream of hazardous waste | Name of the technological stream of hazardous waste | Waste code | Name of the waste | Revised name of waste | Storage of waste | Further treatment of waste |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Maximum allowable at the same time total quantity of waste, t |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |

Table 3: Hazardous wastes authorised for use other than those to be stored and prepared for use

Name of installation

| Dangerous marking of the waste technological stream | Name of the technological stream of hazardous waste | Waste code | Name of the waste | Revised name of waste | Recovery of waste |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Waste recovery activity code (R1-R11) | Design capacity of the installation, t/m |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |

Table 4: Hazardous wastes authorised for disposal other than those intended to be stored and prepared for disposal

Name of installation

| Labelling of the technological stream of hazardous waste | Name of the technological stream of hazardous waste |  | Waste code | Name of the waste | Revised name of waste | Disposal of waste |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Waste disposal activity code (D1–D7, D10) | Design capacity of the installation | Maximum allowable total amount of waste to be disposed of, t/m |
| 1 |  | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 |  | 2 | 3 | 4 | 5 | 6 | 7 | 8 |

Table 5: Hazardous wastes permitted to be prepared for recovery and/or disposal

Name of installation

| Dangerous marking of the waste technological stream | Name of the technological stream of hazardous waste |  | Waste code | Name of the waste | Revised name of waste | Preparation of waste for recovery and/or disposal |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Waste management activity code (D8, D9, D13, D14, R12, S5) | Design capacity of the installation in t/m. |
| 1 |  | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 |  | 2 | 3 | 4 | 5 | 6 | 7 |

Table6:Obligation to have effective financial security where this requirement is applied in compliance with theLaw of the Republic of Lithuania on Waste Management, and other conditions for the treatment (recovery or disposal, including storage and preparation for recovery or disposal) of hazardous waste under the requirements of legal acts regulating environmental protection

| Serial No | Conditions |
| --- | --- |
| 1 | 2 |

___

Rules on the granting, updating and revocation of pollution permits

Annex 3

Appendix 5

SPECIAL PART OF THE PERMIT

EXTRACTION OF WATER FROM SURFACE WATER BODIES

A brief description of the conclusions of the analysis of the operator’s needs in the application to extract water from surface water bodies. A brief description of how water extraction will affect the body of water from which the water is extracted.

Table 1: Information on the surface water body from which the water is to be extracted, the point of abstraction and the permitted amount of water to be extracted.

| Category of water body (river, lake, pond, etc.) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name of body of water |  |  |  |  |  |  |
| Water body identification code |  |  |  |  |  |  |
| Water extraction point No |  |  |  |  |  |  |
| Coordinates of the water extraction point |  |  |  |  |  |  |
| Maximum allowable water abstraction | M3/m | M3/p. | M3/m | M3/p. | M3/m | M3/p. |

Table2:Other conditions for extraction of water from surface water bodies in compliance with the requirements of legal acts regulating environmental protection

| Serial No | Conditions |
| --- | --- |
| 1 | 2 |

Rules on the granting, updating and revocation of pollution permits

Annex 3

Appendix 6

SPECIAL PART OF THE PERMIT

SHIP RECYCLING

Table 1:Allowable capacity of a ship recycling facility

Installation name___

| Ship recycling method(s) | Type and size (length, width, depth, GT) of the largest permissible recycling vessel | Maximum quantity of vessels intended for recycling in the installation to be stored simultaneously (i.e., units, etc.) | Maximum allowable quantity of vessels to be recycled in the installation, units of measurement (i.e., units, etc.) | Maximum annual ship recycling rate (LDT/year) |
| --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 |

Table 2:Wastes from ship recycling authorised to be stored

Installation name ___

| Waste |  |  | Storage of waste | Further treatment of waste | Installation restrictions and conditions, including storage and/or management of hazardous waste |
| --- | --- | --- | --- | --- | --- |
| Code | Title | Revised name | Maximum quantity of waste to be stored at any time, units of measurement (t,m³ or other) |  |  |
| 1 | 2 | 3 | 4 | 5 | 6 |

Table 3:Storage and handling of hazardous substances, including those referred to in Annexes I and/or II to Regulation (EU) No 1257/2013

Installation name ___

| Name of the hazardous substance | Quantity stored simultaneously, units of measurement (t, m³or other) | Authorised storage/storage method | Authorised method of handling | Operating restrictions and conditions of the installation with regard to the storage and handling of hazardous substances |
| --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 |

Table 4:Restrictions on the operation of aship recycling facility and/or other conditions for ship recycling in accordance with the requirements of legal acts regulating environmental protection

| Serial No | Restrictions and conditions |
| --- | --- |
| 1 | 2 |

This part of the permit is valid until___

Rules on the granting, updating and revocation of pollution permits

Annex 3

Appendix 7

SPECIAL PART OF THE PERMIT

ODOR MANAGEMENT

Table 1:Permitted odor dispersion

| Odour source No | Measures for the management/reduction of odour |  |  | Allowed odour emission index OUE/s, OUE/m/s, OUE/m²/s, OUE/m³/s |
| --- | --- | --- | --- | --- |
|  | title | installation location, coordinates, LKS | efficiency, % |  |
| 1 | 2 | 3 | 4 | 5 |

Amendments to the Annex:

A note from the tar. Appendix 7 of Annex 3 to the Rules for Issuing, Replacement and Withdrawal of Pollution Permits shall enter into force on 1 January 2021.

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

Rules on the granting, updating and revocation of pollution permits

Annex 3

Appendix 9

PERMIT NO.......................................ANNEXES

1.

2.

3.

4.

___ ___

(Date of establishment of the list of annexes)

Director of AAA or a person authorised by him ___ ___

(Name)	 (signature)

A. V.

___

Rules on the granting, updating and revocation of pollution permits

## Annex 4 (information)

EXPLANATIONS FOR COMPLETING CERTAIN PARTS OF THE APPLICATION

CHAPTER I

FILLING IN THE PART OF THE SPECIFIC APPLICATION ‘ENVIRONMENTAL AIR POLLUTION MANAGEMENT’

1. For information on stationary sources of pollution (Table 2), pollution into ambient air (Table 3), ambient air pollution treatment plants and pollution prevention measures (Table 4) and pollution into ambient air under unusual/extraordinary operating conditions (Table 5), a separate table shall be completed for each installation.

2. Physical data from stationary sources of pollution shall be provided from the inventory report during the operation of the installation; where an installation plans to carry out an activity – from the documents of environmental impact assessment of the proposed economic activity or technical documents of the installation, where the procedures of environmental impact assessment of the proposed economic activity are not mandatory.

3. The names and codes of pollutants emitted into ambient air, names of ambient air pollutants purification plants shall be entered, the numerical values of emissions shall be rounded in accordance with the inventory rules and the procedure for accounting and reporting of pollutant emissions into ambient air approved by Order No 408 of the Minister of Environment of the Republic of Lithuania on 20 December 1999.

4. For each installation equipped with at least one ambient air pollution treatment plant or subject to measures to prevent ambient air pollution, Table 4 provides information on these treatment plants and on their contaminants. In addition to the name of the installation, details of the purpose and operation of the installation (design degree of cleansing of contaminants caught/decontaminated or the design emission concentration of the purification plant) shall be added.

5. The concentration of emissions to ambient air shall be reported to normal conditions (101,3 kPa and 0 oC).

6. When writing data on pollution into ambient air under unusual (extraordinary) operating conditions, the maximum values that are possible under the unusual (extraordinary) operating conditions specified in the technical regulation of the installation, or other standard (extraordinary) operating conditions, under which, as laid down in paragraph 17.2 of the Regulations, emissions into ambient air will not exceed the ambient air quality standard set for them, shall be specified.

CHAPTER II

COMPLETING THE PART ‘WATER EXTRACTION FROM SURFACE WATER’ OF THE SPECIFIC APPLICATION

Requirements for the completion of Table 1:

1. The identification code of a water body shall be indicated from the Cadastre of Rivers, Lakes and Ponds of the Republic of Lithuania.

2. The size of a lake or pond is determined by the area and the average depth of the water body concerned.

3. Indicate whether the body of water enters the protected area or its buffer zone (yes/no).Where a water body enters a protected area or a buffer zone thereof, a scheme/plan containing the location of the water body in respect of protected areas and buffer zones thereof shall be added.

4. The water extraction location number provided shall correspond to the number shown in the plot plan referred to in point 25.1 of the Rules.

5. For water extraction points, the point at which water is extracted from the water body, the coordinates in the Global Positioning System (GPS).

CHAPTER III

COMPLETION OF THE PART OF THE SPECIFIC APPLICATION ‘MANAGEMENT AND DISCHARGE OF WASTE WATER’

Requirements for the completion of Table 1:

1. The first column shall contain the sequence number of the receiving (surface water body) of the waste water. The acceptance must be marked in the diagram accompanying the application.

2. the second column shall contain a description of the body of surface water into which the waste water is to be discharged or in which the discharge device is to be discharged (in the case of unnamed release into a water body (e.g. land reclamation ditch, pond, etc.), indicate the category of the body: a river, lake, pond, pond, land reclamation ditch, etc.; the code of the water body shall be indicated in compliance with Order No. 594 of 12 December 2001 of the Minister of Environment of the Republic of Lithuania on the Approval of the Classification of Rivers and Ponds of the Republic of Lithuania and Order No 130 of the Minister of Environment of the Republic of Lithuania of 21 March 2003 on the Approval of the Classification of Lakes of the Republic of Lithuania.

3. the third column shall be completed in accordance with the description of the procedure for calculating the environmental water flow rate.

4. Columns 5 to 9 shall be completed when the application is to be accompanied by a calculation of the impact on the receiver of the discharged waste water.

5. the fifth column shall indicate the parameters against which the effect of waste water on the receiving vessel is assessed.

6. Columns 6 to 7 indicate the status of the body of water above the discharger.

7. Columns 8 to 10 shall indicate the results of the permitted impact on a water body at the planned discharge point of the waste water. The calculation/justification of the permissible effects shall accompany the application (may include an extract from the EIA report).

Requirements for the completion of Table 2:

1. in the first column, the sequence number of the receiver of the waste water. The numbering of the receivers shall continue with the last adopted number of the body of water (surface water body) shown in Table 1. The number must coincide with the number at which the waste water discharge/receiver is marked with the application in the object/installation scheme.

2. the second column shall describe where and by what means the discharge/disposal of waste water from the facility/installation is envisaged (e.g. waste water is stored in a reservoir of 300 m³ capacity and is carried out once a month to an urban treatment plant (specifying the carrier, name of the treatment plant and person who operates them); The sewage is infiltrated in the ground by 1000m² underground filtration field; waste water is discharged into sewer networks (identifying the person operating the networks, etc.).

3. the legal basis of the waste water discharge shall be indicated in the third column(e.g., where waste water is planned to be discharged into sewerage networks owned by other persons, the date of the contract of purchase and sale of waste water treatment facilities, duration, number and names of the parties to the agreement; if the waste water is planned to be infiltrated into soil, the date of the signing of the act of acceptance of the filtering device as fit for use, the name of the person who has formed the commission of acceptance, etc. shall be written).

4. information on the accepted load shall be provided if the permissible load of the person receiving the described (intended to be used) waste water is limited (e.g. the limit conditions laid down in the contract of purchase and sale of waste water treatment facilities; set of permissible loads in the design of filtration units, etc.).

5. the fifth column shall specify the parameters according to which the reception of the applicant’s intended discharge of waste water may be limited.

Requirements for the completion of Table 3:

1. in the first column, the number of the effluent or source (the source of the waste water shall be described when the waste water is discharged into the environment or transferred to other persons other than the stationary discharge device (e.g., exit by asenisation machinery, etc.).The number indicated in the table shall correspond to the number by which the discharger or source is marked in the attached plan.

2. the second column shall indicate the coordinate points/places at which the effluent discharges into the environment (e.g. the last well before discharge to the underground filtration plant, end of the discharge into the river, etc.), discharged into sewer networks belonging to other persons, transferred to mobile containers or otherwise taken from the source.

3. in the third column, the number of the accepted person to which the waste water is to be discharged through the described discharge device or from the waste water source described shall be indicated from Table 1 or Table 2.

4. the fourth column shall indicate the type of waste water (industrial, domestic, surface, mixed, slurry, brewers, etc.) and the activity (activities) resulting in waste water intended to be discharged through the discharge device described or from the waste water source described; if subscriber waste water is planned to be released through the issuer, subscriber numbers according to Table 6 are also written (if waste water from different subscribers is planned to be released through different issuers).

5. the fifth column shall indicate the type of discharge or waste water source (e.g. quay, vagina, bottom, surface filtration, underground filtration, discharge into sewer networks, storage tank, etc.) and technical data (distance from the shore, depth, diameter, capacity, etc.).

6. the sixth column describes the location of the discharge device, e.g. the position of the discharge device in relation to the course (right bank, left bank, centre of the river), point of connection (street name, etc.).

7. the maximum amount of effluent to be discharged in the seventh and eighth columns may not exceed the maximum hydraulic load of the accepted maximum hydraulic load specified in Tables 1 or 2.

Requirements for the completion of Table 4:

1. in the first column, the release device/source number according to Table 3.

2. The second column shall indicate the pollutants for which an authorisation is required under the legislation in force.

3. in the third column, the maximum expected concentration of the pollutant in the instantaneous or average daily effluent sample prior to treatment.

4. in the fourth column, the maximum expected annual average concentration of the pollutant in the effluent before treatment. Columns 3 to 6 are to be completed if the waste water is to be treated before discharge from the facility/installation.

5. in the sixth column, the maximum allowable concentration (MRLs) of the therapeutic agent established/calculated in the instantaneous or average daily sample of waste water in accordance with the legislation in force and the Regulations (depending on the acceptance, nature of the activity carried out, etc.).The application must be accompanied by a justification for setting the MRLs.

6. in the seventh column, the acceptable concentration of the pollutant in the instantaneous or average daily effluent sample (MRL/LLC) desired to be entered in the permit. For a pollutant to be set in the authorisation in excess of the MRL in column 7 (i.e. when a LL is requested), the application must be accompanied by an appropriate justification in accordance with the requirements of the Rules.

7. in the eighth column, the maximum allowable annual concentration (MRLs) for a pollutant established/calculated in accordance with the legislation in force and the Regulations (depending on the acceptance, the nature of the activity, etc.).The application must be accompanied by a justification for setting the MRLs.

8. in the ninth column, the average annual concentration (MRL/LLC) of the pollutant desired to be set in the permit. In order to establish a permitted level of a pollutant in the authorisation above the MRL in column 8 (i.e. when a TRL is requested), the application must be accompanied by an appropriate justification in accordance with the requirements of the Rules.

9. in the tenth column, the maximum daily emission limit (DLT) set/calculated in accordance with the legislation in force and the Regulations (depending on the acceptance, nature of the activity carried out, etc.).The application must be accompanied by a calculation of the DLT.

10. in the eleventh column, the permitted daily release of the pollutant (contamination allowed) is requested in the permit. In order to determine the permitted contamination in the permit in excess of that indicated in column 11 DLT (when an LLT is requested), the application must be accompanied by an appropriate justification in accordance with the requirements of the Regulations and regulations.

11. in the twelfth column, the maximum annual emission limit (DLT) (depending on the acceptance, the nature of the activity carried out, etc.) established/calculated in accordance with the applicable legislation and the Regulations. The application must be accompanied by a calculation of the DLT.

12. in the thirteenth column, the annual discharge limit (permitted pollution) is requested in the permit. In order to determine the permitted contamination in the permit in excess of that indicated in column 13 DLT (when an LLT is requested), the application must be accompanied by an appropriate justification in accordance with the requirements of the Regulations and regulations.

Requirements for the completion of Table 5:

1.the first column shall contain the number of the waste water or pollution reduction measure;

2. in the second column, the number of the waste water source/discharger from Table 3, during which the measure to reduce the impact of discharges is applied.

3. the third column briefly describes the measure for the reduction of waste water (e.g. the water circulation system of a car wash, etc.) or the reduction of pollution (industrial, domestic, surface waste water treatment plants, etc.) and its purpose (e.g. to remove petroleum products and drowning substances from the surface waste water, reduce the amount of waste water, etc.).

4. in the fourth column, the date of introduction of the measure.

5. Columns 5 to 7 contain the design features of the instrument as specified in the design documentation. Columns 5, 6 record design indicators for reducing waste water and pollution related to the parameters requested in the permit (e.g. plant efficiency: m³/d, circulatory flow: l/s; design contamination of waste water entering the treatment plant according to OBD, N, P, petroleum products, total Cr, etc. – mg/l, t/d. residual contamination according to OBD, N, P, petroleum products, total Cr, etc. – mg/l; clean-up efficiency – per cent or similar.

Requirements for the completion of Table 6:

1. The fourth column refers to pollutants subject to legislation for discharge of waste water.

Requirements for the completion of Table 7:

1. In the second column, the numbering of subscribers shall continue with the last subscriber number specified in Table 6.

2. the fourth column shall indicate the pollutants the discharges of which, in accordance with the legislation in force, are subject to authorisation for the treatment of the surface effluent.

CHAPTER IV

COMPLETION OF THE PART ‘SHIP RECYCLING’ OF THE SPECIFIC APPLICATION

Table1: Information on the ship recycling facility. This table shall be completed for installations carrying out ship recycling activities.

Requirements for the completion of Table 1:

1. The first column shall indicate the ship recycling method used in the installation, where several methods are used, the methods for recycling different types and sizes of ships shall be listed, indicating the type and size of ships to be used for recycling.

2. The second column shall indicate the type and size (length, width, depth, GT (gross tonnage)) of the largest ship likely to be recycled in the installation. The dimensions of the ship shall be determined in accordance with the International Convention on Tonnage Measurement of Ships, 1969.

3. The third column indicates the maximum quantity of vessels to be stored simultaneously in the installation intended for recycling.

4. The fourth column indicates the maximum quantity of vessels to be recycled in the installation at the same time.

5. The fifth column shall indicate the planned maximum annual ship recycling capacity for the installation calculated in accordance with the provisions of Article 32(1)(a) of Regulation (EU) No 1257/2013.

Table2: Information on waste resulting from ship recycling (applies to installations where the waste generated at the place where it is produced is kept for less than the temporary storage period laid down in the Law of the Republic of Lithuania on Waste Management and which does not carry out activities related to the treatment of waste).

Requirements for the completion of Table 2:

1. The first column shall indicate the waste code in accordance with Annex 1 to the Waste Management Rules.

2. The second column shall indicate the name of the waste in accordance with Annex 1 to the Waste Management Rules.

3. The name of the revised waste shall be indicated in the third column in accordance with Annex 1 to the Waste Management Rules.

4. The fourth column indicates the maximum quantity of waste to be stored in the installation at the same time.

5.The fifth column shall indicate the planned further waste treatment activities. Where an intermediate waste treatment activity is indicated, the final waste treatment activity in accordance with Annex 2 to the Waste Management Rules must also be indicated in accordance with the priority order of waste prevention and management referred to in Article 3 of the Law of the Republic of Lithuania on Waste Management.

Table3: Information on hazardous substances contained in the structure and/or equipment of the ship to be recycled, including those specified in Annexes I and/or II to Regulation (EU) No 1257/2013. To be completed when the installation processes vessels of 500 GT or more. In the case of installations processing ships of less than 500 GT, this table shall be completed in the case of ships with a design and/or equipment containing dangerous substances intended to be recycled.

Requirements for the completion of Table 3:

1. The name (nomenclature) of the hazardous substance shall be indicated in the first column.

2. The second column shall indicate the estimated quantity of hazardous substances stored simultaneously.

3. The third and fourth columns indicate how these hazardous substances (if they do not become waste after recycling) will be stored and handled.

CHAPTER V

FILLING IN THE SPECIFIC PART OF THE APPLICATION “WASTE TREATMENT (RECOVERY OR DISPOSAL, INCLUDING STORAGE AND PREPARATION FOR RECOVERY OR DISPOSAL)”

1.Table1 shall be completed by operators providing for the storage of waste destined for further recovery and/or disposal (the term ‘storage of waste’ shall be understood as defined in the Law on Waste Management of the Republic of Lithuania).The table below lists the wastes to be stored and waste resulting from the treatment of waste:

1.1. in column ‘Code’, a six-digit waste code from the list of waste referred to in Annex 1 to the Waste Management Rules, e.g. 15 01 03;

1.2. the name of the waste shall be entered under the heading ‘Name’ in accordance with Annex 1 to the Waste Management Rules, e.g. wood packaging;

1.3. in the ‘Revised name’ column, a detailed description of the waste is indicated, e.g. the revised name of the waste may be pallets, wooden boxes, etc.;

1.4. in the column ‘Waste management activity code (R13 and/or D15)’, the waste storage activity code (R13 and/or D15) should be indicated in accordance with Annex 2 to the Waste Management Rules;

1.5. under “Maximum amount of waste, including waste generated during management, to be stored at the same time, t” shall indicate the maximum amount to be regarded as total in tonnes of all planned waste and waste resulting from the treatment. In the case of hazardous waste, the total amount of waste from each technological stream of hazardous waste;

1.6. under “Planned further treatment of waste” refers to the planned further treatment of waste (code and name) in accordance with Annex 2 to the Waste Management Rules. Where an intermediate treatment activity is indicated, the final treatment activity (code and name) of the waste shall also be indicated in accordance with Annex 2 to the Waste Management Rules;

1.7. in the column “Notification of technological stream of hazardous waste” the marking of the technological stream of hazardous waste is indicated in accordance with Annex 1 of the Rules for Licensing of Hazardous Waste Management approved by Order No. 684 of the Minister of Environment of the Republic of Lithuania of 19 December 2003 on the Approval of Rules for Licensing of the Management of Hazardous Waste (hereinafter referred to as the "Regulations for Licensing of Hazardous Waste Management"), Annex 1, e.g. TS —01;

1.8. under “Name of the technological stream of hazardous waste” refers to the name of the technological stream of hazardous waste in accordance with Annex 1 to the Rules for Licensing of Hazardous Waste, e.g. waste containing polychlorinated biphenyls (PCBs).

2. For the purposes of Table2, operators intending to store hazardous waste generated outside the waste management site for more than six months and non-hazardous waste for more than one year prior to collection (S8):

2.1. in column ‘Code’, a six-digit waste code from the list of waste referred to in Annex 1 to the Waste Management Rules, e.g. 15 01 03;

2.2. the name of the waste shall be entered under the heading ‘Name’ in accordance with Annex 1 to the Waste Management Rules, e.g. wood packaging;

2.3. the ‘Revised name’ column gives a detailed description of the waste, e.g. the revised name of the waste may be pallets, wooden boxes, etc.;

2.4. under ‘Maximum total quantity of waste to be stored at the same time, t’ shall indicate the maximum total quantity of non-hazardous waste to be considered at the place where it is produced, expressed in tonnes. In the case of hazardous waste, the total amount of waste from each technological stream of hazardous waste;

2.5. under “Planned further treatment of waste” refers to the planned further treatment of waste (code and name) in accordance with Annex 2 to the Waste Management Rules. Where an intermediate treatment activity is indicated, the final treatment activity (code and name) of the waste shall also be indicated in accordance with Annex 2 to the Waste Management Rules;

2.6. the labelling of the technological stream of hazardous waste is indicated in accordance with Annex 1 to the Rules for Licensing of Hazardous Waste, e.g. TS-01.

2.7. under “Name of the technological stream of hazardous waste” refers to the name of the technological stream of hazardous waste in accordance with Annex 1 to the Rules for Licensing of Hazardous Waste, e.g. waste containing polychlorinated biphenyls (PCBs).

3. Table 3 shall be completed by undertakings intending to recover (other than storage and pre-treatment of waste intended for recovery):

3.1. in column ‘Code’, a six-digit waste code from the listof wastereferred to in Annex 1 to the Waste Management Rules, e.g. 15 01 03;

3.2. the name of the waste shall be entered under the heading ‘Name’ in accordance with Annex 1 to the Waste Management Rules, e.g. wood packaging;

3.3. the ‘Revised name’ column gives a detailed description of the waste, e.g. the revised name of the waste may be pallets, wooden boxes, etc.;

3.4. in the column ‘Waste recovery activity code (R1–R11)’, the waste recovery activity code (R1–R11) in accordance with Annex 2 to the Waste Management Rules;

3.5. the design capacity of the installation, t/m, the design capacity of the waste recovery facility, expressed in tonnes per year;

3.6. under “Planned further treatment of waste” refers to the planned further treatment of waste (code and name) in accordance with Annex 2 to the Waste Management Rules.Where an intermediate treatment activity is indicated, the final treatment activity (code and name) of the waste shall also be indicated in accordance with Annex 2 to the Waste Management Rules;

3.7. the labelling of the technological stream of hazardous waste is indicated in accordance with Annex 1 to the Rules for Licensing of Hazardous Waste, e.g. TS-01.

3.8. under the heading ‘Name of the technological stream of hazardous waste’, the name of the technological stream of hazardous waste is given in accordance with Annex 1 to the Rules for Licensing of Hazardous Waste, e.g. waste containing polychlorinated biphenyls (PCBs).

4. For the purpose of Table 4, undertakings intending to dispose of waste (other than storage and pre-treatment of waste destined for disposal) shall be completed:

4.1. in column ‘Code’, a six-digit waste code from the list of waste referred to in Annex 1 to the Waste Management Rules, e.g. 15 01 03;

4.2. the name of the waste shall be entered under the heading ‘Name’ in accordance with Annex 1 to the Waste Management Rules, e.g. wood packaging;

4.3. the ‘Revised name’ column gives a detailed description of the waste, e.g. the revised name of the waste may be pallets, wooden boxes, etc.;

4.4. under waste disposal activity code (D1–D7), D10, code of the waste disposal activity (D1–D7), D10) in accordance with Annex 2 to the Waste Management Rules;

4.5. under ‘Design capacity of an installation’, an indication of the design capacity of the installation. Operators operating landfills shall indicate the design capacity of the installation over the entire life of the landfill in tonnes. Operators of waste incineration plants or waste co-incineration plants shall indicate the capacity of the waste incineration plant or waste co-incineration plant in tonnes per year;

4.6. under “Maximum total amount of waste to be disposed of, t/m” indicates the maximum total (total volume of all waste disposed of) expected to be disposed of in tonnes/year. In the case of hazardous waste, the total amount of waste disposed of by technological stream of hazardous waste in tonnes per year;

4.7. under “Technological flow marking of hazardous waste” shall include the marking of the technological stream of hazardous waste in accordance with Annex 1 to the Rules for Licensing of Hazardous Waste, e.g. TS-01;

4.8. under “Name of the technological stream of hazardous waste” refers to the name of the technological stream of hazardous waste in accordance with Annex 1 to the Rules for Licensing of Hazardous Waste, e.g. waste containing polychlorinated biphenyls (PCBs).

5. Table 5 shall be completed by undertakings intending to prepare waste for recovery and/or disposal:

5.1. in column ‘Code’, a six-digit waste code from the list of waste referred to in Annex 1 to the Waste Management Rules, e.g. 15 01 03;

5.2. the name of the waste shall be entered under the heading ‘Name’ in accordance with Annex 1 to the Waste Management Rules, e.g. wood packaging;

5.3. in the ‘Revised name’ column, a detailed description of the waste, e.g. the revised name of the waste may be pallets, wooden boxes, etc.;

5.4. in the column ‘Code of waste management operations (D8, D9, D13, D14, R12, S5)’ the waste management activity code (D8, D9, D13, D14, R12, S5) is indicated in accordance with Annex 2 to the Waste Management Rules;

5.5. under ‘Installation’ design capacity, t/m, the design capacity of the installation, expressed in tonnes per year, for preparing for recovery and/or disposal;

5.6. the labelling of the technological stream of hazardous waste is indicated in accordance with Annex 1 to the Rules for Licensing of Hazardous Waste, e.g. TS-01.

5.7. under “Name of the technological stream of hazardous waste” refers to the name of the technological stream of hazardous waste in accordance with Annex 1 to the Rules for Licensing of Hazardous Waste, e.g. waste containing polychlorinated biphenyls (PCBs).

CHAPTER VI

COMPLETION OF THE PART OF THE SPECIFIC APPLICATION ‘MANAGEMENT OF SMELLS’

1. For activities carried out in an installation, physical data from stationary sources of odour (Table 1) shall be shown from the inventory report: in the case of the activity planned to be carried out in the installation – from documents of environmental impact assessment of the proposed economic activity, technical documents of the installation, documents of public health impact assessment of the proposed economic activity, design documentation of a construction works where environmental impact assessment procedures of the proposed economic activity are not mandatory. The column “Coffee emission indicator” of Table1 shall be filled out of the documents of environmental impact assessment of the proposed economic activity, technical documents of the installation, documents of public health impact assessment of the proposed economic activity, construction projects and/or according to data of stationary sources of pollution measurement. If the odour emission index is calculated, it must be ensured that the odour concentration at the nearest receptor (after adjusting the calculated odour emission index) does not exceed the specified odour concentration limit.

2. For an installation subject to odour management/reduction measures, the information on the odor management/reducing measures applied in the sources of odour, their efficiency (in percent) after the introduction and application of the specified odour control/reduction measures are presented in the form set out in Table 2. The efficiency column of ‘Flood management (reduction) measures’ is to be filled in, taking into account the effectiveness of odour reduction measures as specified in BAT, Lithuanian or other legislation or recommendations, research data, technical specifications for installations, etc.

3. Table 3 ‘Efficacy efficiency at closest receptors’ provides information on the concentration of odour (OUE/m³) at the nearest sensitive receptor (OUE/m³) at the nearest sensitive receptor (OUE/m³) in a given format and taking into account the explanations given in paragraphs 1 and 2. In Table3, column 'Determined odour concentration (OUE/m³)at the nearest sensitive receptor is to be filled in according to the results of odour modelling/calculation using the data in the column 'OUE/s, OUE/m/s, OUE/m²/s, OUE/m³/s, OUE/m²/s, OUE/m³/s’ in Table 2.

4. When laboratory tests are performed to determine the odour concentration modelling and/or odour emission at a stationary source of pollution, air samples shall be taken to determine the odour concentration and laboratory tests of odour concentration shall be carried out using an accredited method in accordance with the Lithuanian standard LST EN 13725: 2004+AC: 2006 "Air quality. Determination of odour concentration by dynamic olfactometry’.

5.Modelling of odour scattering is performed by selecting a calculation model according to the recommendations for the calculation of the models for the calculation of the dissemination of pollutants for the assessment of the impact of agricultural activities on ambient air, approved by Order No. AV-200 of the Director of the Environmental Protection Agency of 9 December 2008 “On the Approval of the Recommendations for the Calculation of the Models for the Calculation of the Models of the Dissemination of Contaminants of Farm Activities for Environmental Air Assessment”.

___

Rules on the granting, updating and revocation of pollution permits

Annex 5 (information)

PROCEDURE FOR FILLING IN THE TITLE SHEET (COVER) OF A PERMIT

|  | Permit shall be issued or renewed in accordance with the procedure laid down in the Rules | The IPPC permit issued in accordance with the IPPC Rules of 2002 shall be amended in accordance with the procedure laid down in the Ordinance on the Approval of Rules for Issuing, Replacement and Cancellation of Pollution Authorisations |
| --- | --- | --- |
| (example of pollution permit) ___ (name of issuing or amending authority (in capitals)) PERMIT No___ (Legal personal number) (name and address of the object of the economic activity) (operator, address, telephone, fax, e-mail address) The authorisation shall consist of: 1. Special part(s): ...................................... 2. Registration number of the IPPC permit in force prior to the renewal of the permit (was issued), the name of the regional environmental protection department which issued it, the dates of issue, renewal and/or correction (if any): ....................... 3. Additives to the authorisation Issued on..................................................... Amended by...................................................... The Director or a person authorised by him ___ ___ (Name) (signature) A. V. | Enter the full name of AAA The issuing of a new No AAA in accordance with the procedure laid down in the AAA and the replacement shall retain the same No. Identification of any special parts in force after the granting or modification of the authorisation Not to be completed Indicate the date of issuance of the permit to the AAA Individual line shows the date of each AAA change E.g. Amended on1 January 2015; Amended on1 July 2015; Amended on1 October 2015; (etc) | Enter the full name of AAA A registration number consisting of the No of the RAAD and the new AAA No shall be assigned. E.g. No XXX/YYY All parts of the Special Authorisation issued or modified in accordance with the requirements of the Regulations shall be indicated Indicate the name of the RAAD which issued the IPPC permit, the date of issue of the IPPC permit, the registration number and the dates of any renewals and/or corrections made to the IPPC permit) Not to be completed Indicate the date on which the IPPC permit was replaced by a permit and any subsequent AAA change in accordance with the requirements of the Regulations E.g. Amended on1 January 2015; Amended on1 July 2015; Amended on1 October 2015; (etc) |

The amendments are as follows:

1.Ministry of Environment of the Republic of Lithuania, Order

No D1-1049, 2014-12-19 published TAR 2014-12-22, i.e. 2014-20406

Amending Order No D1-259 of the Minister of Environment of the Republic of Lithuania of 6 March 2014 on the Approval of Rules for the Issuance, Replacement and Revocation of Pollution Permits

2.Ministry of Environment of the Republic of Lithuania, Order

No D1-252, 2015-03-30 published TAR 2015-03-31, i.e. 2015-04852

Amending Order No D1-259 of the Minister of Environment of 6 March 2014 on the Approval of the Rules for Issuing, Replacement and Revocation of Pollution Authorisations

3.Ministry of Environment of the Republic of Lithuania, Order

No D1-754, 2015-10-23 published TAR 2015-10-26, i.e. 2015-16737

Amending Order No D1-259 of the Minister of Environment of 6 March 2014 on the Approval of the Rules for Issuing, Replacement and Revocation of Pollution Authorisations

4.Ministry of Environment of the Republic of Lithuania, Order

No D1-270, 2016-04-21 published TAR 2016-04-28, i.e. 2016-10626

Amending Order No D1-259 of the Minister of Environment of the Republic of Lithuania of 6 March 2014 on the Approval of Rules for the Issuance, Replacement and Revocation of Pollution Permits

5.Ministry of Environment of the Republic of Lithuania, Order

No D1-995, 2017-12-14 published TAR 2017-12-18, i.e. 2017-20260

Amending Order No D1-259 of the Minister of Environment of the Republic of Lithuania of 6 March 2014 on the Approval of Rules for the Issuance, Replacement and Revocation of Pollution Permits

6.Ministry of Environment of the Republic of Lithuania, Order

No D1-401, 2017-05-11 published TAR 2017-05-12, i.e. 2017-08065

Amending Order No D1-259 of the Minister of Environment of the Republic of Lithuania of 6 March 2014 on the Approval of Rules for the Issuance, Replacement and Revocation of Pollution Permits

7.Ministry of Environment of the Republic of Lithuania, Order

No D1-29, 2018-01-16 published TAR 2018-01-16, i.e. 2018-00674

Amending Order No D1-259 of the Minister of Environment of 6 March 2014 on the Approval of the Rules for Issuing, Replacement and Revocation of Pollution Authorisations

8.Ministry of Environment of the Republic of Lithuania, Order

No D1-536, 2018-06-19 published TAR 2018-06-20, i.e. 2018-10130

Amending Order No D1-259 of the Minister of Environment of the Republic of Lithuania of 6 March 2014 on the Approval of Rules for the Issuance, Replacement and Revocation of Pollution Permits

9.Ministry of Environment of the Republic of Lithuania, Order

No D1-357, 2019-06-11 published TAR 2019-06-11, i.e. 2019-09454

Amending Order No D1-259 of the Minister of Environment of the Republic of Lithuania of 6 March 2014 on the Approval of Rules for the Issuance, Replacement and Revocation of Pollution Permits

10.Ministry of Environment of the Republic of Lithuania, Order

No D1-425, 2020-07-16 published TAR 2020-07-16, i.e. 2020-15850

Amending Order No D1-259 of the Minister of Environment of the Republic of Lithuania of 6 March 2014 on the Approval of Rules for the Issuance, Replacement and Revocation of Pollution Permits